# 國立臺北大學資訊工程學系

Department of Computer Science & Information Engineering

National Taipei University

碩士論文

Master Thesis

指導教授：陳裕賢 博士

Advisor：Dr. Yuh-Shyan Chen

低軌道多波束衛星網路之節能型換手決策：基於角度感知多鯰魚式多目標強化學習

Energy-Efficient Handover Decision for LEO Multi-Beam Satellite Networks Based on Angle-Aware Multi-Catfish MODQN

研究生：吳柏宏

Student：Bo-Hung, Wu

中華民國 115 年 7 月

July, 2026

# Abstract

Low Earth Orbit (LEO) satellite networks offer wide coverage and low latency, but the high speed of the satellites causes rapidly changing beam coverage, so handovers are frequent and hard to decide. The existing Multi-Objective Deep Q-Learning Network (MODQN) models this problem with three reward terms — throughput, handover cost, and load balancing — yet the throughput reward reflects little of how beam off-axis angle and interference affect energy efficiency. Each user also takes its own arg max without seeing the choices of others, so users tend to point at the same beam and over-concentrate, users on one beam share its bandwidth, so concentration directly lowers each of their rates, while the learning side never sees what the other users choose at that step. This baseline is extended into a Multi-Catfish Reinforcement Learning (MCRL) framework. On the reward side, throughput is replaced with angle-aware energy efficiency, and transmit power is determined directly by link angle gain: each uninterrupted served physical-link segment starts at the segment-start power p0, and only consecutive service of the same physical link uses the previous-step power and angle-gain recurrence. When off-axis angle reduces beam gain, the recurrence can raise RF power within a continuing segment, while realized SINR, throughput, and system consumption jointly determine energy efficiency, handover, outage, unserved steps, re-entry, and episode reset restart the segment. At training time, the main side and the catfish side are each a three-objective MODQN: their three Q-networks map one-to-one to energy efficiency, handover, and load balancing, while the three catfish-side networks share one environment trajectory and one replay buffer. Experience shaping uses energy-efficiency stratification, asymmetric discounting, and periodic mixed-batch intervention, reward shaping adds a paired competitive term only to the catfish-side energy-efficiency reward. Both operate only during training, the deployment rule is unchanged.

低軌衛星（LEO）網路覆蓋廣、延遲較低，但衛星高速移動使波束覆蓋快速變化，換手因而頻繁且不易決策。既有的多目標深度 Q 學習網路（Multi-Objective Deep Q-Learning Network, MODQN）以吞吐量、換手成本與負載平衡三個獎勵項建模此問題，但吞吐量獎勵未反映偏軸角與干擾對能量效率的影響。每位使用者又各自取最大值、看不到彼此的選擇，容易一起指向同一波束而過度集中；同一波束上的使用者均分頻寬，集中會直接壓低每個人的速率，而學習端從訓練到部署都看不到其他使用者當步的選擇。以該方法為基準，擴充為鯰魚多目標強化式學習（Multi-Catfish Reinforcement Learning, MCRL）框架。獎勵端把吞吐量改為角度感知能量效率，並由鏈路角度增益直接決定發射功率：每一段 uninterrupted served physical link 以固定的段起始功率 p0 起始，只有連續同一實體鏈路時才依前一步功率與角度增益比遞推。偏軸角降低波束增益時，同一連續服務段的遞推功率可能上升，並與實際 SINR、吞吐量及系統耗能共同改變能量效率；handover、outage、unserved、re-entry 與 episode reset 都重新起始。訓練時，主代理與鯰魚代理各是一組三目標 MODQN：兩端的三個 Q 網路均與能量效率、換手及負載平衡一一對應，鯰魚代理三網路則共用一條環境軌跡與一個經驗池。經驗塑形採能效分層、非對稱折扣與週期性混合批次介入，獎勵塑形只在鯰魚代理能效獎勵加入配對競爭項，兩者都只在訓練期作用，部署選法不變。

Keywords: Low Earth Orbit Satellite, Multi-Beam Network, Handover Decision, Multi-Objective Reinforcement Learning, MODQN, Multi-Catfish, Load Balancing, Energy Efficiency

關鍵字：低軌衛星、多波束網路、換手決策、多目標強化式學習、MODQN、Multi-Catfish、負載平衡、能量效率

# 1. Introduction

In recent years, mobile communications have advanced toward sixth-generation systems and non-terrestrial networks. Low Earth Orbit (LEO) satellites provide broad coverage and low propagation delay, and can serve remote areas, maritime regions, and airspace where ground infrastructure is difficult to deploy, which is why they have drawn attention. However, LEO satellites orbit at high speed, causing the available satellites and beams to change continuously, making connection maintenance and handover decisions more difficult.

近年來行動通訊朝第六代行動通訊與非地面網路發展。低軌衛星能提供大範圍覆蓋與較低傳播延遲，可服務偏遠地區、海上與空中等難以部署地面基礎建設的環境，因而受到重視。但低軌衛星高速繞行地球，使用者可連接的衛星與波束隨時間不斷改變，連線維持與換手決策也因此更加困難。

In multi-beam LEO satellite systems, a single satellite serves different geographic regions using multiple beams, and users may require handover between beams or satellites due to satellite movement, coverage boundary changes, or degrading service quality. Using signal strength alone for beam selection may cause excessive load on certain beams, prioritizing handover reduction may leave users on progressively degrading connections, focusing on load balancing may sacrifice transmission quality for some users. Therefore, multi-beam LEO handover should not rely on a single metric but must simultaneously consider multiple objectives such as throughput, handover cost, and load balancing.

在多波束低軌衛星系統中，一顆衛星以多個波束服務不同地理區域，使用者可能因衛星移動、波束覆蓋邊界改變或服務品質下降，而需在不同波束或不同衛星間換手。若只依訊號強度選波束，可能造成部分波束負載過高；只追求減少換手，可能讓使用者停留在逐漸變差的連線；只顧負載平衡，又可能犧牲部分使用者的傳輸品質。因此，低軌衛星的多波束換手不宜只用單一指標，而需同時考慮吞吐量、換手成本與負載平衡等多個目標。

To address this type of dynamic decision problem, existing studies have approached LEO satellite handover through deep reinforcement learning and multi-objective learning, respectively emphasizing access delay and collisions, and throughput and handover cost [1], [2]. Among them, Sun et al. formulated multi-beam LEO satellite handover as a multi-objective deep-reinforcement-learning problem using a Multi-Objective Deep Q-Learning Network (MODQN) [2].

為了處理這類動態決策問題，既有研究已從深度強化式學習與多目標學習等方向處理低軌衛星換手問題，分別關注接取延遲與碰撞，以及吞吐量與換手成本等面向 [1], [2]。其中，Sun 等人提出的多目標深度 Q 學習網路（Multi-Objective Deep Q-Learning Network, MODQN）將低軌衛星多波束換手表達為多目標深度強化式學習問題 [2]。

Energy-aware, beam-gain, and spectrum-coexistence studies further show that link geometry, power cost, and interference affect handover and energy evaluation [3], [4], [5]. Handover work under dynamic propagation emphasizes channel state and load [6], while beam management under stochastic traffic and time-varying topology highlights the tradeoff among service quality, load, and system resources [7]. These studies, however, mostly address individual parts of the problem, a framework that jointly connects off-axis angle, transmit power determined directly by angle gain, and MODQN-based multi-objective training for multi-beam LEO satellite handover remains absent.

另一方面，能源感知、波束增益與頻譜共存相關研究顯示，鏈路幾何、功率成本與干擾都會影響換手及能源評估 [3], [4], [5]；動態傳播條件下的換手研究強調通道狀態與負載 [6]，隨機流量和時變拓樸下的波束管理則凸顯服務品質、負載與系統資源之間的取捨 [7]。然而，前述研究多從個別面向處理問題，尚缺少一個將偏軸角、由角度增益直接決定的發射功率，以及 MODQN 多目標訓練整合於低軌衛星多波束換手的框架。

This study therefore extends the MODQN baseline into Multi-Catfish Reinforcement Learning (MCRL), modifying the reward and the training procedure to address energy efficiency and beam concentration. The main contributions are as follows:

因此，本文以 MODQN 為基準，擴充為鯰魚多目標強化式學習（Multi-Catfish Reinforcement Learning, MCRL），並針對能量效率與波束集中問題調整獎勵與訓練方式。主要貢獻如下：

- MCRL retains MODQN’s multi-objective reinforcement-learning architecture and replaces the first reward term with angle-aware energy efficiency. Each uninterrupted served physical-link segment starts at the segment-start power p0, and consecutive service of the same link applies the previous-step power and angle-gain recurrence, the same realized power then determines interference, throughput, and system power consumption, with no cap, clip, or projection in the physical formula.

- MCRL retains the deployment rule in which each user independently selects its maximum-valued action. Training adds experience shaping and reward shaping, and both act only during training, the input representation and the selection rule at deployment are the same as in MODQN.

- MCRL 保留 MODQN 的多目標強化式學習架構，把第一個獎勵項由吞吐量改為角度感知能量效率。每個 uninterrupted served physical-link segment 以段起始功率 p0 起始，連續同一實體鏈路時由前一步功率與角度增益比遞推；同一個實際功率再決定干擾、吞吐量與系統耗能，公式不加 cap、clip 或 projection。

- MCRL 保留部署時逐使用者取最大值的選擇規則。訓練端加入經驗塑形與獎勵塑形，兩者只在訓練期間作用。部署時的輸入表示與選法都與 MODQN 相同。

The remaining chapters are organized as follows. Chapter 2 reviews related work on LEO satellite handover, multi-beam resource management, energy efficiency, and reinforcement learning. Chapter 3 describes the system model and problem formulation. Chapter 4 introduces the MCRL method. Chapter 5 describes the simulation settings and evaluation procedure. Chapter 6 summarizes the study and discusses limitations and future directions.

其餘章節安排如下。第二章回顧低軌衛星換手、多波束資源管理、能量效率與強化式學習的相關研究。第三章說明系統模型與問題形式化。第四章介紹 MCRL 方法。第五章說明模擬設定與評估方式。第六章總結研究內容，並說明研究限制與未來可能方向。

# 2. Background

This chapter reviews LEO satellite handover, multi-objective reinforcement learning, catfish competitive learning, and capacity-constrained decisions, then positions the problem addressed in this study.

本章先回顧低軌衛星換手、多目標強化式學習、鯰魚競爭學習與容量受限決策，再說明本研究的問題定位。

## 2.1 Related Work

LEO satellite handover involves high-speed satellite motion, changing beam coverage, user connection quality, and system load. Existing studies have applied deep reinforcement learning directly to handover decisions. Lee et al. [1] learned handover protocols that reduce access delay and collisions. Learning through interaction with the environment helps these methods adapt to time-varying communication conditions.

低軌衛星換手問題涉及衛星高速移動、波束覆蓋變化、使用者連線品質與系統負載等因素。既有研究首先將深度強化式學習用於換手決策。Lee 等人 [1] 以深度強化式學習學習換手協定，以降低接取延遲與碰撞情形。這類方法透過與環境互動學習決策，較能因應隨時間變化的通訊環境。

As energy costs receive attention, handover decisions have also begun incorporating energy considerations. Ntabeni et al. [3] proposed energy-aware Q-learning that jointly considers signal quality, handover frequency, and energy efficiency. Chen et al. [4] proposed a joint LEO handover and fast beam switching (HOBS) algorithm that uses historical signal-quality and beam-index information to narrow the search range and reduce beam-training latency, it also accounts for signal-to-interference-plus-noise ratio, beam transmit power, beam gain, beam switching, and both intra-satellite and inter-satellite interference. Mendonça et al. [5] considered off-axis antenna loss and interference management in spectrum-coexistence handover, showing that beam angle and interference affect handover decisions. Liu et al. [6] used multi-agent deep reinforcement learning with a three-state Markov model of dynamic propagation conditions, allowing decisions to consider channel state, load, and handover performance. The channel, interference, and power models in Chapter 3 draw on these treatments of signal quality and beam power.

隨著能源成本受到重視，換手決策也開始納入能量考量。Ntabeni 等人 [3] 提出能量感知 Q 學習，把訊號品質、換手次數與能量效率一併納入。Chen 等人 [4] 提出聯合低軌衛星換手與快速波束切換（HOBS）演算法，利用歷史訊號品質與波束索引縮小搜尋範圍，以降低波束訓練延遲；該方法同時考量訊號干擾雜訊比、波束發射功率、波束增益與波束切換，並把同衛星與跨衛星干擾納入訊號品質計算。Mendonça 等人 [5] 在頻譜共存的換手情境中考量偏軸天線損失與干擾管理，說明波束角度與干擾對換手決策具有影響。Liu 等人 [6] 則以多代理深度強化式學習處理換手，以三態 Markov 模型描述動態傳播條件，使決策能同時考慮通道狀態、負載與換手表現。第三章的通道、干擾與功率模型參考了上述研究對訊號品質與波束功率的處理方式。

Multi-objective reinforcement learning addresses the trade-offs among transmission performance, handover cost, and load distribution. Sun et al. [2] formulated LEO multi-beam handover as a multi-objective deep-reinforcement-learning problem in MODQN, using throughput, handover cost, and load balancing as three reward terms and learning their action values with three parallel Q-networks. Because it addresses the same handover setting and objectives, MODQN is the primary baseline for this study. Song et al. [8] also incorporated throughput, handover frequency, and load balancing into multi-objective handover. Tajmajer [9] used decision values for modular scalarization, whereas Basaklar et al. [10] processed the objectives separately during learning rather than combining them into a single scalar at the outset. MCRL retains MODQN’s three objective networks and linear scalarization, while changing the first reward and the training procedure.

多目標強化式學習著重於換手決策中的多項取捨。Sun 等人 [2] 建立的 MODQN 把低軌衛星的多波束換手設計為多目標深度強化式學習問題，以吞吐量、換手成本與負載平衡為三個獎勵項，並用三個並行的 Q 網路各自學習一個目標的動作價值。由於同樣關注低軌衛星的多波束換手、且需同時處理多個目標，MODQN 是本方法最主要的基準。Song 等人 [8] 也以深度強化式學習將吞吐量、換手次數與負載平衡納入多目標換手。Tajmajer [9] 以決策值進行模組化純量化；Basaklar 等人 [10] 則讓不同目標在學習過程中分別處理，而非在一開始合成單一純量。MCRL 沿用 MODQN 的三個目標網路與線性純量化，改動集中在第一個獎勵的定義與訓練方式。

The MODQN baseline backbone used in this study is shown in Fig. 2-1. Three Q-networks estimate action values for energy efficiency, handover, and load balancing, which are scalarized by weights Ω and selected with ϵ-greedy exploration. The environment returns a three-dimensional reward and the next state, after transitions enter replay, target networks construct temporal-difference targets for updating the three online networks. The first objective in the figure uses the angle-aware energy efficiency defined in this study, the additional MCRL training mechanisms are described in Chapter 4.

本研究採用的 MODQN 基準骨幹如圖 2-1。三個 Q 網路分別估計能量效率、換手與負載平衡的動作價值，再以權重 Ω 純量化並透過 ϵ-greedy 選擇動作。環境回傳三維獎勵與下一狀態，轉移存入回放池後，以目標網路建立時間差分目標並更新三個線上網路。圖中第一個目標採用本研究的角度感知能量效率；MCRL 額外的訓練機制於第四章說明。

Fig. 2-1 MODQN baseline used in this study

Fig. 2-1: MODQN baseline backbone used in this study. The three objective Q-networks share state input and a feasible-action mask, then scalarize with weights Ω to select actions. Environment transitions enter replay, and the three online/target network pairs update with their respective objective rewards. The first objective shows the angle-aware energy efficiency adopted here, which differs from the original MODQN throughput objective, no additional MCRL training-time shaping is included.

The selection and use of training experience also influence the learned policy. Schaul et al. [11] proposed prioritized experience replay, adjusting sampling probabilities from temporal-difference errors so that more informative transitions are used more often. Ke [12] proposed Catfish Deep Reinforcement Learning (CDRL), which introduces a catfish role during training to promote exploration and experience use without directly producing deployment decisions. CDRL was originally applied to energy-efficient control in RIS-aided communications. This study extends that auxiliary-training approach to multi-objective handover: both the main and catfish agents are three-objective MODQNs, and the three networks within each agent correspond to the three objectives. In entity-based multi-agent reinforcement learning, a user, satellite, or beam is chosen as the agent unit and multiple agents make decisions jointly. The two MCRL agents instead serve as training roles for the same handover problem, neither controls one particular network entity. The catfish agent supplies additional experience and competitive signals during training, while the main agent continues to learn from its own experience. Chapter 4 describes these mechanisms.

訓練經驗的選取與使用也會影響學到的策略。Schaul 等人 [11] 提出的優先經驗回放依時間差分誤差調整取樣機率，讓資訊量較高的轉移更常被使用。Ke [12] 提出的鯰魚效應深度強化式學習（CDRL）則在主代理旁加入鯰魚角色，於訓練中促進探索與經驗利用，而不直接輸出部署決策。CDRL 原本用於 RIS 輔助通訊的節能控制；本研究將此輔助訓練方式延伸到多目標換手，使主代理與鯰魚代理都採三目標 MODQN，每個代理內的三個網路分別對應三個目標。一般按網路實體建模的多代理強化式學習，會依問題設定把使用者、衛星或波束當作代理單位，讓多個代理共同決策。MCRL 的兩個代理則是同一換手問題中的訓練角色，不分別控制某個使用者、衛星或波束。鯰魚代理在訓練期間提供額外經驗與競爭訊號，主代理仍根據自己的經驗更新。第四章將說明這些訓練機制。

Capacity and resource constraints are another important issue in multi-beam systems. Zhu et al. [7] studied beam management under stochastic traffic arrivals and time-varying topology, balancing service quality, beam load, and system resources. For reinforcement-learning problems with constraints, Calvo-Fullana et al. [13] added constraint information to the state, and Agorio et al. [14] applied the same approach to multi-agent assignment. Holder et al. [15] used reinforcement learning for satellite assignment under limited resources, while Ye et al. [16] studied load-aware user association in cellular networks. The training-shaping strategies in Chapter 4 draw on these approaches.

容量與資源限制也是多波束系統的重要問題。Zhu 等人 [7] 在隨機流量到達與時變拓樸的條件下探討多波束低軌衛星網路的波束管理，嘗試在服務品質、波束負載與系統資源之間取得平衡。在以強化式學習處理約束方面，Calvo-Fullana 等人 [13] 將約束資訊加入狀態，Agorio 等人 [14] 也將此作法用於多代理指派問題。Holder 等人 [15] 以強化式學習處理有限資源下的衛星指派，Ye 等人 [16] 則在蜂巢式網路中研究兼顧負載平衡的使用者關聯。第四章的訓練塑形策略以這些約束處理方法為參考。

## 2.2 Motivation

Taken together, the studies reviewed in Section 2.1 reveal three issues that have not been addressed jointly. First, the first reward in existing multi-objective handover methods is often still throughput-centered, while energy-aware work less often connects angle gain, off-axis-induced power changes, interference, realized throughput, and system energy consumption in a single energy-efficiency chain. Second, MODQN’s three objectives generally use the same learning workflow, while prior catfish-style auxiliary learning has mainly been studied in single-objective problems, objective-wise auxiliary training has not yet been defined. Third, independent per-user selection cannot directly see the other users’ choices or the resulting beam congestion at the same time step. Existing constraint approaches often change the deployment-time assignment procedure or add constraint information to the state, fewer address it while retaining the original deployment rule.

綜合第 2.1 節，本研究聚焦三個尚未被同時處理的問題。第一，既有多目標換手方法的第一個獎勵往往仍以吞吐量為主；能量感知研究則較少把鏈路角度增益、偏軸角造成的功率變化、干擾、實際吞吐量與系統能耗串成同一條能效計算鏈。第二，MODQN 的三個目標通常採用相同的學習流程，而既有鯰魚式輔助學習多設定在單一目標問題，尚未定義如何讓輔助訓練按不同目標分工。第三，每位使用者獨立選擇時，無法直接看見同一時間步其他使用者的選擇與各波束的壅塞程度；既有約束處理常改變部署時的指派程序，或將約束資訊加入狀態，較少在保留原本部署選法的前提下處理這個問題。

MCRL addresses these problems through two design choices. First, it replaces the first reward with each user’s additive contribution to angle-aware system energy efficiency, so that the link-quality and power costs caused by off-axis angle enter the same objective. Second, in Multi-Catfish training, both the main and catfish sides use three-objective MODQNs, whose three Q-networks correspond to energy efficiency, handover, and load balancing. The training process includes experience shaping and reward shaping: experience shaping uses energy-efficiency stratification, asymmetric discounting, and periodic mixed-batch intervention to change the experiences used during training, reward shaping adds a same-state competitive comparison only to the catfish side’s first objective, thereby aligning the auxiliary training with the corresponding objectives. The two shaping strategies act only during training. At deployment, only the main agent is retained and each user still takes its own argmax.

本研究以兩類設計回應上述問題。第一，將第一個獎勵改為角度感知系統能量效率的逐使用者可加貢獻，讓偏軸角造成的鏈路品質與功率代價進入同一個目標。第二，在 Multi-Catfish 訓練中，主代理與鯰魚代理都採三目標 MODQN，兩組各自的三個 Q 網路分別對應能量效率、換手與負載平衡。訓練過程包含經驗塑形與獎勵塑形：經驗塑形透過能效分層、非對稱折扣與週期性混合批次介入改變訓練時取用的經驗；獎勵塑形只在鯰魚代理的第一個目標加入同狀態的競爭比較，使輔助訓練有明確的目標對應。兩個塑形策略只在訓練期作用，部署時只保留主代理，每位使用者仍各自取最大值。

A larger off-axis angle reduces beam gain and can lower the received signal power, SINR, and throughput. Within a continuing served segment, a lower current gain than the previous gain can raise RF power under the previous-step recurrence and may increase power-amplifier consumption. A new segment does not retain inactive-link power and instead restarts at p0, the recurrence has no power cap, clip, or projection. Section 3.1.3 gives the complete model, and Section 3.2 defines the formal reward.

偏軸角增大時，波束增益降低，接收訊號與 SINR 可能下降，因而降低吞吐量；在同一連續服務段，前一步增益較高而目前增益降低時，previous-step recurrence 可能提高目前 RF power，並改變功率放大器耗能。新 segment 不沿用 inactive link 的功率，而是重新從 p0 開始；recurrence 不含 power cap、clip 或 projection。完整模型於第 3.1.3 節說明，正式獎勵於第 3.2 節定義。

# 3. Preliminaries

This chapter first explains how a user selects a service beam from the visible beams. It then introduces the signal quality, throughput, and power consumption of the selected link. Finally, these quantities are organized into the three objectives of energy efficiency, handover cost, and load balancing.

本章先說明使用者如何從可見波束中選擇服務波束，再介紹這條連線的訊號品質、吞吐量與耗能。最後將這些量整理成能量效率、換手成本與負載平衡三個目標。

## 3.1 System Model

### 3.1.1 Network Model

Consider a downlink multi-beam LEO satellite communication system. Because the satellites keep moving, the satellites and beams visible to each user change over time. Time is discretized into successive steps of length Δt, and the system selects one available service beam for each user at every step, the value of Δt is given in Section 5.1. The overall scenario is shown in Fig. 3-1.

考慮一個多波束低軌衛星下行通訊系統。低軌衛星持續移動，因此使用者可見的衛星與波束會隨時間改變。將時間離散為依序排列、長度為 Δt 的時間步，系統在每一步為每位使用者選擇一個可用的服務波束；Δt 的設定列於第 5.1 節。整體場景如圖 3-1 所示。

Fig. 3-1 LEO multi-beam satellite system model

Fig. 3-1: System model of a multi-beam LEO satellite network. Each user selects a service beam from the candidates provided by visible satellites. Beam direction affects signal quality, and users selecting the same beam share its resources. The system must also account for beam interference and handover cost, a beam that no user selects is not activated.

#### Users, Satellites, and Beams

Let U={1,…,U}, S={1,…,S}, and V={1,…,V} denote the user, satellite, and beam-index sets, respectively. The symbols u, s, v, and t denote user, satellite, beam, and time-step indices.

以 U={1,…,U}、S={1,…,S} 與 V={1,…,V} 分別表示使用者、衛星與波束索引集合；u、s、v 與 t 則表示使用者、衛星、波束與時間步索引。

Let xu,s,v(t) represent the realized connection. When xu,s,v(t)=1, user u is served by beam v of satellite s, otherwise it is 0. Each user can connect to at most one beam, so

令 xu,s,v(t) 表示實際連線結果。當 xu,s,v(t)=1 時，使用者 u 由衛星 s 的波束 v 服務；否則為 0。每位使用者最多連上一個波束，因此

xu,s,v(t)∈{0,1},  s∈S​v∈V​xu,s,v(t)≤1.(3.1)

Let zs,v(t) indicate whether beam (s,v) is active. For every u∈U, s∈S, and v∈V, a user can connect only to an active beam, so

以 zs,v(t) 表示波束 (s,v) 是否啟用。對任意 u∈U、s∈S 與 v∈V，使用者只能連到已啟用的波束，因此

zs,v(t)∈{0,1},  xu,s,v(t)≤zs,v(t).(3.2)

#### Beam Activation and Load

The number of users actually served by beam (s,v) is

波束 (s,v) 實際服務的使用者數量為

Us,v(t)=u∈U​xu,s,v(t).(3.3)

This study activates only beams with connected users, so beam activation follows directly from the connection result:

本研究只啟用有使用者連接的波束，波束的啟用狀態因此由連線結果直接決定：

zs,v(t)=1,Us,v(t)>0,0,Us,v(t)=0,  ∀s∈S, v∈V.(3.4)

A beam with no users transmits no power and creates no interference. This study sets no separate upper bound on how many beams one satellite may activate at the same time.

沒有使用者的波束不發射功率，也不產生干擾。本研究不另設每顆衛星可同時啟用的波束數上限。

### 3.1.2 Geometry and Channel Model

In LEO satellite communications, geometry determines both the distance between a user and a satellite and the user’s direction relative to a beam center. Following non-terrestrial network channel models and LEO satellite research [17], [18], the slant range between user u and satellite s is defined as in (3.5):

在低軌衛星通訊中，幾何關係同時決定使用者與衛星的距離，以及使用者相對於波束中心的方向。參考非地面網路通道模型與低軌衛星相關研究 [17], [18]，定義使用者 u 與衛星 s 之間的斜距如式 (3.5)：

du,s,v(t)=RE2sin2αu,s(t)+hs2+2REhs−REsinαu,s(t).(3.5)

Here, αu,s(t) is the elevation angle of the user relative to the satellite, RE is the Earth’s radius, and hs is the satellite altitude. The index v records the slant range at the candidate-link level, its value may be identical across beams on the same satellite. The slant range is one component of the linear link-power factor used below, which does not directly carry the wanted-link angular pattern.

其中 αu,s(t) 為使用者相對於衛星的仰角，RE 為地球半徑，hs 為衛星高度。v 將斜距記為候選波束鏈路層量；對同一顆衛星的不同波束，它的數值可以相同。斜距是後續線性鏈路功率因子的一部分，該因子不直接承載 wanted-link 的角度型樣。

The off-axis angle describes the angular difference between the user’s direction and the beam-center direction. A multibeam LEO model defines this angle through the arccosine of two direction vectors [19, Eq. (5)], so the off-axis angle of user u relative to beam (s,v) is defined as in (3.6):

偏軸角描述使用者方向與波束中心方向之間的角度差。多波束低軌衛星模型以兩個方向向量的夾角定義偏軸角 [19, Eq. (5)]，因此使用者 u 相對於波束 (s,v) 的偏軸角定義如式 (3.6)：

θu,s,v(t)=arccosvu,s,v(t)⋅ru,s,v(t)∥vu,s,v(t)∥∥ru,s,v(t)∥.(3.6)

Here, vu,s,v(t) is the beam-center direction recorded for candidate link (u,s,v), and ru,s,v(t) is the satellite-to-user direction recorded for the same link. The three indices mark link-level data ownership. v may have the same value across users of one beam, and r may have the same value across beams of one user-satellite pair. The off-axis angle is the point at which geometric information enters the energy-efficiency chain.

其中，vu,s,v(t) 表示候選鏈路 (u,s,v) 的波束中心方向向量，ru,s,v(t) 表示同一候選鏈路由衛星指向使用者的方向向量。兩者以三下標標示資料所有權；同一波束的 v 可在不同使用者間取相同值，同一使用者與衛星的 r 也可在不同波束間取相同值。偏軸角是本文把幾何資訊導入能量效率的入口。

Following the angular pattern commonly used for multi-beam satellites and adopted in HOBS [4], [20], the transmit gain is retained only as a function of the off-axis angle:

沿用多波束衛星常用、亦見於 HOBS 的角度型樣 [4], [20]，發射端增益只保留其對偏軸角的依賴：

GT​θ,θ3dB,  GT​0,θ3dB=G0.(3.7)

Here, G0 is the beam-center gain. To keep the tunable antenna parameters traceable to the simulator, the main text uses a one-layer normalized-pattern expansion:

其中 G0 為波束中心增益。為了讓正文的可調天線參數與模擬器一一對應，採用一層 normalized pattern 展開：

GT​θ,θ3dB=G0F​θ,θ3dB,  GT​0,θ3dB=G0,  F​0,θ3dB=1.(3.8)

The angular pattern adopts the J1/J3 pattern of HOBS Eq. (3) [4], the angle argument and pattern are

角度型樣採用 HOBS 式 (3) [4] 的 J1/J3 型樣，角度參數與 pattern 為

μ​θ,θ3dB=2.07123sinθsinθ3dB/2,  F​θ,θ3dB=J1​μ​θ,θ3dB2μ​θ,θ3dB+36J3​μ​θ,θ3dBμ​θ,θ3dB32.(3.9)

Here J1(⋅) and J3(⋅) are the Bessel functions of the first kind of orders 1 and 3, and 2.07123 is that equation’s fixed angle argument, the same value that appears in other multibeam-satellite work. F is naturally unity at θ=0 (1/4+3/4), so no extra boresight normalization constant is required, however J1(μ)/(2μ) and 36J3(μ)/μ3 are 0/0 at μ=0, so the limit is taken directly whenever |μ|<ϵμ, with ϵμ given in Section 5.1. θ3dB is a fixed antenna parameter, but it appears on the right-hand side of the definition of μ and, through μ, sets the shape of F and GT, it is therefore listed as an argument of all three, so that the left-hand side of every definition covers the quantities that appear on its right. G0 is different: it is a multiplicative scale constant that does not change the shape of the pattern and is therefore not an argument. From Eq. (3.10) onward the same function is applied at a specific link’s off-axis angle, so it is written with the fixed parameter explicitly as GT(θu,s,v,θ3dB). This layer does not introduce a three-index gain variable and does not add a pattern selector.

其中 J1(⋅) 與 J3(⋅) 分別為第一類貝索函數的一階與三階，2.07123 是該式的固定角度參數，同一數值亦見於其他多波束衛星文獻；F 在 θ=0 自然等於 1（1/4+3/4），因此不需要額外的 boresight 正規化常數；但 J1(μ)/(2μ) 與 36J3(μ)/μ3 在 μ=0 是 0/0，數值上以 |μ|<ϵμ 時直接取該極限值處理，ϵμ 的設定列於第 5.1 節。θ3dB 雖是固定的天線參數，但它出現在 μ 定義式的右側，並經由 μ 決定 F 與 GT 的形狀，因此列為三者的引數：每一條定義式的等號左側都必須涵蓋右側出現的量。G0 不同，它只是乘性的尺度常數，不改變型樣形狀，故不列入引數。式 (3.10) 之後的公式在特定鏈路的偏軸角上套用同一個函數，並明寫固定參數為 GT(θu,s,v,θ3dB)。這一層不引入帶三下標的增益變數，也不設型樣選擇器。

This study does not adopt the G0=40 dBi of Table I of HOBS [4]. The aperture 10c/fc, the gain of 40 dBi and the beamwidth listed there are mutually inconsistent: under either dimensionally valid reading, 40 dBi requires an aperture efficiency of η=2.53 (radius reading) or η=10.13 (diameter reading), both greater than one. The aperture is therefore re-derived from the beamwidth, D=1.0275λ/θ3dB=17.7λ, with the Ka-band aperture efficiencies of 0.639–0.645 back-derived from 3GPP TR 38.821 [22], this gives G0=2000 (33.0 dBi). The value is listed in Section 5.1. Throughout this thesis θ3dB denotes the full half-power beamwidth, consistent with the cell radius Rb=hstan(θ3dB/2) in Section 5.1, whereas HOBS Eq. (3) normalizes by the one-sided half-power angle, Eq. (3.9) therefore substitutes θ3dB/2. The constant 2.07123 is calibrated to exactly that point: F=1/2 at μ=2.07123.

本研究不沿用 HOBS [4] Table I 的 G0=40 dBi。該表列出的孔徑 10c/fc、增益 40 dBi 與波束寬三值互斥；在量綱合法的兩種讀法下，40 dBi 分別要求孔徑效率 η=2.53（半徑讀法）或 η=10.13（直徑讀法），均大於 1。本研究改由波束寬一致的孔徑重導：D=1.0275λ/θ3dB=17.7λ；取 3GPP TR 38.821 [22] 各列反推的 Ka 頻段孔徑效率 0.639–0.645，得 G0=2000（33.0 dBi）。數值列於第 5.1 節。本研究的 θ3dB 一律指全半功率波束寬，與第 5.1 節的格半徑 Rb=hstan(θ3dB/2) 一致；HOBS 式 (3) 以單邊半功率角為分母，故式 (3.9) 代入 θ3dB/2。常數 2.07123 正校準於此點：μ=2.07123 時 F=1/2。

Let Hu,s,v(t) denote the linear link-power factor that does not directly carry the wanted-link angular pattern. To trace it to the channel implementation, expand H as

令 Hu,s,v(t) 表示不直接承載 wanted-link 角度型樣的線性鏈路功率因子。為了與通道實作對應，H 展開為

Hu,s,v(t)=10−Lu,s,v(t)10Gu,s,vR(t),(3.10a)

where

其中

Lu,s,v(t)=Lf​du,s,v(t),fc+Lg​αu,s(t)+Lc​αu,s(t)+Ls​αu,s(t).(3.10b)

The four losses correspond to L=Lfs+Lg+Lsc+Lsf of HOBS Eq. (1), this thesis writes them as L=Lf+Lg+Lc+Ls to keep subscripts single-lettered: Lf is the free-space path loss, Lg the atmospheric gaseous absorption, Lc the scintillation and Ls the shadow fading, the last three vary with elevation, and their models and values are given in Section 5.1, all four are dB terms converted to a common linear scale before entering H, and Gu,s,vR(t) is the linear receive gain. These four terms are exactly the loss set of HOBS Eq. (1), the public expansion adds nothing below them. Scan loss, NLoS clutter, and other implementation-layer channel corrections are absorbed into Hu,s,v(t), take no public symbol, and are not tunable parameters.

四項損耗對應 HOBS 式 (1) 的 L=Lfs+Lg+Lsc+Lsf，本文為維持單字母下標記為 L=Lf+Lg+Lc+Ls：Lf 為自由空間路徑損耗、Lg 為大氣氣體吸收、Lc 為閃爍、Ls 為遮蔽衰落，後三項皆隨仰角變化，其模型與數值列於第 5.1 節；四項皆以 dB 表示，代入 H 前轉成線性尺度；Gu,s,vR(t) 是接收端線性增益。這四項即 HOBS 式 (1) 的損耗集合，公開展開不再往下加項；掃描損耗、NLoS clutter 等實作層通道修正併入 Hu,s,v(t)，不取得公開符號，也不是可調參數。

Gu,s,vR(t) is written as a boresight gain minus an off-axis loss. Let θu,sR(t) be the angle in degrees between the user’s receive-antenna pointing direction and the direction of satellite s. Then

Gu,s,vR(t) 以軸心增益扣除離軸損耗表示。令 θu,sR(t) 為使用者接收天線指向與第 s 顆衛星方向之間的夾角（以度為單位），則

Gu,s,vR(t)=min​max​AR−BRlog10θu,sR(t), GR,min, GR,max.(3.10c)

AR, BR, and the floor GR,min are taken from the ITU-R S.465-6 earth-station reference radiation pattern [21], whose stated range of 2 to 31 GHz covers the fc used here. The three are not independent settings: the recommendation gives AR−BRlog10θR for θminR≤θR<48∘ and GR,min for 48∘≤θR≤180∘, and the two branches join continuously at 48∘. The boresight gain GR,max is the derated value adopted for a comparable 0.6 m user terminal in an LEO handover setting [5], about 4.7 dB below the ideal gain of that aperture. The recommendation defines the envelope only for θR≥θminR (here θminR≈2.498∘ at fc), this work extends the same envelope into θR<θminR up to the GR,max cap, which is a modelling simplification outside the scope of the recommendation.

AR、BR 與下限 GR,min 取自 ITU-R S.465-6 的地球站參考輻射型樣 [21]，其適用範圍為 2 至 31 GHz，涵蓋本文的 fc。三者並非各自獨立設定：該建議書以 AR−BRlog10θR 描述 θminR≤θR<48∘，以 GR,min 描述 48∘≤θR≤180∘，兩段在 48∘ 連續銜接。軸心增益 GR,max 取自低軌換手情境下同級 0.6 m 使用者終端所採的降額值 [5]，較該孔徑的理想增益保守約 4.7 dB。該建議書只在 θR≥θminR 定義此包絡（本文終端在 fc 下 θminR≈2.498∘）；本文於 θR<θminR 延用同一包絡至 GR,max 截止，此為建模上的簡化，不在該建議書的規定範圍內。

The two wanted-link linear power-gain factors are still used directly as

wanted-link 的兩個線性功率增益因子仍直接相乘為

Hu,s,v(t)GT​θu,s,v,θ3dB.(3.10)

Here, GT directly carries the off-axis-angle dependence, while Hu,s,v(t) preserves the physical meaning of the remaining channel factors — the four losses of Eq. (3.10b), the receive gain, and Rician small-scale fading, whose K-factor KR is a scenario calibration listed in Section 5.1. Both are linear power-scale factors, not complex channel amplitudes. The thesis, the simplified symbol table, and the / legacy simulator frontend share this one public expansion layer that maps to simulator controls and runtime fields, only layout may differ, not formulas, symbols, or editable parameters. The public expansion is capped at the depth the source paper itself shows: the four loss terms of HOBS Eq. (1) and a single J1/J3 angular-pattern layer whose two-term structure follows HOBS Eq. (3). Deeper antenna measurement, Bessel-approximation, and channel-calibration details are outside the public formula layer. No intermediate channel symbol distinguished only by letter case is introduced. Note also that the MODQN Gi,l,v is a channel gain whose role corresponds to Hu,s,v here, semantically different from the antenna gains GT and GR, the two same-letter symbols are not merged.

其中 GT 直接承載偏軸角，Hu,s,v(t) 保留其餘通道因素的物理意義——包含式 (3.10b) 的四項損耗、接收增益，以及萊斯小尺度衰落（其 K 因子 KR 屬情境校準，列於第 5.1 節）；兩者都是線性功率尺度的因子，不表示複數通道振幅。正文與簡化符號表、/ legacy simulator 前端共用這個能和控制項／runtime 欄位對應的一層公開展開；三者只允許排版不同，不允許公式、符號或可調參數不同。公開展開的深度上限即 HOBS 正文所示的深度：式 (1) 的四項損耗，以及式 (3) 那一層的單一 J1/J3 角度型樣結構。更深的天線量測、Bessel 近似誤差與通道校準細節不屬於公開主公式層級；不另建立只靠大小寫區分的中間通道符號。另需注意 MODQN 的 Gi,l,v 是通道增益，其角色對應本文的 Hu,s,v，與此處的天線增益 GT、GR 語意不同，不可同名合併。

### 3.1.3 Power, SINR and Throughput Model

The following keeps only actual quantities for each user–satellite–beam link. pu,s,v denotes actual RF transmit power, it is not obtained by inverting a target SINR, minimum rate, or lagged interference. If the implementation needs measured, predicted, or cached provenance, that information belongs in field metadata rather than in a second physical SINR.

以下只保留每一條 UE–衛星–波束鏈路的實際量。pu,s,v 表示實際 RF 發射功率，不再由目標 SINR、最低速率或 lagged interference 反推；若程式需要標記 measured、predicted 或 cached provenance，該資訊放在資料欄位 metadata，不另建立第二個物理 SINR。

Let τu,s,v be the start time of the current uninterrupted served physical-link (u,s,v) segment. Episode reset, handover, outage, an unserved step, and re-entry all break continuity, a new segment does not retain inactive-link power and starts at p0. Only consecutive service of the same physical link uses the previous-step recurrence:

令 τu,s,v 為目前 uninterrupted served physical link (u,s,v) segment 的起始時間步。episode reset、handover、outage、unserved 與 re-entry 都會結束 continuity；新 segment 不保留 inactive-link cache，並以 p0 起始。只有同一實體鏈路在前後兩步都被服務時，角度感知功率才使用 previous-step recurrence：

This is an angle-aware power rule defined by this study, not an equation from the original MODQN paper. The modelling condition is to keep the product of transmit power and transmit gain constant within one continuous served segment: pu,s,v(t,θu,s,v(t),θ3dB)GT(θu,s,v(t),θ3dB)=pu,s,v(t−1,θu,s,v(t−1),θ3dB)GT(θu,s,v(t−1),θ3dB). Solving for the current power gives the equation below. It compensates only the transmit-side angular gain, it does not claim that the full SINR or throughput remains constant.

這是本研究自行定義的角度感知功率規則，不是原始 MODQN 的公式。其建模條件是同一連續服務段內維持發射功率與發射增益的乘積： pu,s,v(t,θu,s,v(t),θ3dB)GT(θu,s,v(t),θ3dB)=pu,s,v(t−1,θu,s,v(t−1),θ3dB)GT(θu,s,v(t−1),θ3dB)。 把目前功率解出後，就得到下式；它只補償發射端角度增益，並不宣稱完整 SINR 或 throughput 固定。

pu,s,v​t,θu,s,v(t),θ3dB=pu,s,v​t−1,θu,s,v(t−1),θ3dBGT​θu,s,v(t−1),θ3dBGT​θu,s,v(t),θ3dB,  xu,s,v(t−1)=xu,s,v(t)=1, GT​θu,s,v(t),θ3dB>0,(3.11)

The segment-start condition and its within-segment closed form are:

segment 起始與 segment 內的 closed form 為

pu,s,v​τu,s,v,θu,s,v(τu,s,v),θ3dB=p0,  pu,s,v​t,θu,s,v(t),θ3dB=p0 GT​θu,s,v(τu,s,v),θ3dBGT​θu,s,v(t),θ3dB.(3.12)

Equation (3.12) is valid only when every step from τu,s,v through t remains the same served physical link with positive gain, it is the telescoped identity of Eq. (3.11), not runtime state across an event. The recurrence expresses the previous-step angle-gain effect on link RF power and introduces no target SINR, no required-power inversion, no beam/satellite cap, no PA clamp, and no min/max projection.

式 (3.12) 只有在 τu,s,v 到 t 的每一步都維持同一 served physical link 且角度增益為正時成立；它是式 (3.11) 的 telescoped identity，不是跨事件的 runtime state。這個關係表達前一步角度增益如何遞推實際鏈路 RF power，不引入目標 SINR、需求功率反推、beam／satellite cap、PA clamp 或 min／max 投影。

Define the system angle state θ(t) as the collection of off-axis angles θu,s,v(t) over every u∈U, s∈S, v∈V, and let Iu,s,v(t,θu,s,v,θ3dB) denote the total co-channel interference relative to the wanted link.

在系統層定義全體角度狀態 θ(t)——即每一個 u∈U, s∈S, v∈V 的偏軸角 θu,s,v(t) 所組成的集合——並令 Iu,s,v(t,θu,s,v,θ3dB) 表示相對於 wanted link 的總同頻干擾。

The beams use three-colour frequency reuse. Let cs,v∈{0,1,2} be the frequency colour of beam (s,v), assigned from the axial coordinates (qs,v,rs,v) of its ground cell on the hexagonal lattice as cs,v=(qs,v−rs,v) mod 3. The rule guarantees that any two adjacent cells carry different colours: beams of the same colour share one sub-band, and beams of different colours are orthogonal. The bandwidth available to one beam is therefore Bw=Bsys/3, and interference arrives only from activated beams of the same colour as the serving beam. The cell layout and the axial coordinates are environment settings, given in Section 5.1.

波束以三色頻率重用配置。令 cs,v∈{0,1,2} 為波束 (s,v) 的頻率顏色，依其地面蜂巢在六角格上的軸向座標 (qs,v,rs,v) 指派為 cs,v=(qs,v−rs,v) mod 3。此規則使六角格上任兩個相鄰蜂巢必為異色：同色波束共用同一子頻帶，異色波束彼此正交。因此單一波束的可用頻寬為 Bw=Bsys/3，而干擾只來自與服務波束同色且已啟用的其他波束。蜂巢排列與各波束的軸向座標屬環境設定，於第 5.1 節說明。

An activated beam radiates at a single power regardless of how many users it carries. Following the aggregation convention of the runtime, let ps,v​t,θ,θ3dB=maxu∈U: xu,s,v(t)=1pu,s,v​t,θu,s,v(t),θ3dB be that beam’s transmit power. The left-hand side retains the system angle state and the fixed beamwidth because the right-hand side takes a maximum across all served links. Following HOBS and the multi-beam LEO satellite system model [4], [20], the interference splits into same-satellite and cross-satellite terms, given in Eq. (3.12a) and Eq. (3.12b):

一支已啟用的波束以單一功率發射，不論其上載有幾位使用者；沿用執行端的聚合慣例，令 ps,v​t,θ,θ3dB=maxu∈U: xu,s,v(t)=1pu,s,v​t,θu,s,v(t),θ3dB 為該波束的發射功率。左側保留全系統角度狀態與固定波束寬參數，因為右側會在所有已服務鏈路中取最大值。參考 HOBS 與多波束低軌道衛星系統模型 [4], [20]，將干擾分為同衛星干擾與跨衛星干擾，分別如式 (3.12a) 與式 (3.12b)：

Iu,s,vintra(t,θu,s,v,θ3dB)=v′∈V, v′≠vcs,v′=cs,v​zs,v′(t) ps,v′​t,θ,θ3dB GT​θu,s,v′(t),θ3dBHu,s,v′(t),(3.12a)

Iu,s,vinter(t,θu,s,v,θ3dB)=s′∈S, s′≠s​ v′∈Vcs′,v′=cs,v​zs′,v′(t) ps′,v′​t,θ,θ3dB GT​θu,s′,v′(t),θ3dBHu,s′,v′(t),(3.12b)

so that Iu,s,v(t,θu,s,v,θ3dB)=Iu,s,vintra(t,θu,s,v,θ3dB)+Iu,s,vinter(t,θu,s,v,θ3dB). Every term uses the interfering beam’s own off-axis angle and linear link factor, because different beams reach the same user through different path losses and gains, the sums are gated by the activation indicator z and are not weighted by how many users a beam carries.

且 Iu,s,v(t,θu,s,v,θ3dB)=Iu,s,vintra(t,θu,s,v,θ3dB)+Iu,s,vinter(t,θu,s,v,θ3dB)。每一項都以該干擾波束自己的偏軸角與線性鏈路因子計算，因為不同波束到同一位使用者的路徑損耗與增益都不同；求和以啟用指示 z 為門檻，而不是以其上載有幾位使用者為權重。

The noise power is σ2=kBTBw, where kB is the Boltzmann constant and T=Ta+T010NF/10−1 is the receiver system temperature. The antenna temperature Ta, the receiver noise figure NF and the noise-figure reference temperature T0 take the VSAT values of 3GPP TR 38.821 [22] and are listed in Section 5.1. The noise integrates over the whole beam bandwidth and therefore does not scale with the beam’s load. The unique link SINR is

雜訊功率為 σ2=kBTBw，其中 kB 為波茲曼常數，T=Ta+T010NF/10−1 為接收系統溫度。Ta 為天線溫度、NF 為接收機雜訊指數、T0 為雜訊指數的參考溫度，數值取自 3GPP TR 38.821 的 VSAT 設定 [22]，列於第 5.1 節。雜訊在整支波束的頻寬上積分，因此與該波束的載量無關。唯一的鏈路 SINR 為

γu,s,v​t,θu,s,v,θ3dB=pu,s,v​t,θu,s,v,θ3dBHu,s,v(t)GT​θu,s,v,θ3dBIu,s,v​t,θu,s,v,θ3dB+σ2,(3.13)

with Iu,s,v(t,θu,s,v,θ3dB)+σ2>0. This symbol is used for both baseline-link calculation and angle-aware power substitution, no estimated, required, target, or serving-index-nested SINR is introduced.

其中 Iu,s,v(t,θu,s,v,θ3dB)+σ2>0。這個符號同時用於基線鏈路計算與角度感知功率替換；不另定義 estimated、required、target 或帶 serving-index 巢狀下標的 SINR。

Multiplexing within a beam is time division. The Us,v(t) users on a beam take turns using the available bandwidth. The link throughput is [2], [4]

波束內採時分多工，同一波束上的 Us,v(t) 位使用者依序使用可用頻寬。因此鏈路 throughput 為 [2], [4]

Ru,s,v​t,θu,s,v,θ3dB=BwUs,v(t)log2​1+γu,s,v​t,θu,s,v,θ3dB,  Us,v(t)>0.(3.14)

Bw is the available bandwidth of one beam, for a single-user illustration, set Us,v=1 without creating a second rate or SINR formula. The connection variable xu,s,v(t) selects the active service link and does not create a user-level SINR or throughput alias.

Bw 是單一波束可用頻寬；單一使用者展示時只令 Us,v=1，不另建立第二套 rate 或 SINR 公式。連線變數 xu,s,v(t) 只負責選取實際服務鏈路，不另建立 user-level 的 SINR 或 throughput 別名。

A beam is driven by a single power amplifier, so supply-side power is a per-beam quantity rather than a per-link one. Let ξs,v​t,θ,θ3dB>0 be that beam’s effective conversion efficiency from RF power to supply-side consumption. Then

一支波束由單一功率放大器驅動，因此電源端功率是逐波束而非逐鏈路的量。令 ξs,v​t,θ,θ3dB>0 為該波束由 RF power 到電源端消耗的有效轉換效率，則

Ps,vp​t,θ,θ3dB=ps,v​t,θ,θ3dBξs,v​t,θ,θ3dB.(3.15)

Radiated power is not the power actually consumed. A beam’s RF signal is produced by a power amplifier, and the ratio of its RF output to the DC power it draws is the ξ of Eq. (3.15). An amplifier is more efficient near saturation but less linear there, so it is operated a fixed distance below saturation, that distance is the output back-off BO, expressed in dB, and the saturated power is psat=pmax10BO/10, where pmax is the per-beam operating limit after back-off. Following the power-efficiency relation of the classical class-B amplifier [23], the back-off region is approximated by a load-dependent square-root curve, this is an engineering approximation for system-level analysis, not a full circuit model:

發射出去的功率不等於實際耗掉的功率。波束的射頻訊號由功率放大器產生，其射頻輸出與所取用直流功率之比即為式 (3.15) 的 ξ。放大器接近飽和點時效率較高但較不線性，因此實務上退離飽和點一段距離運作，該距離稱為輸出回退 BO（以 dB 表示）：飽和功率為 psat=pmax10BO/10，其中 pmax 是回退後的每波束操作上限。參考傳統類 B 功率放大器的功率與效率關係 [23]，以負載相依的平方根曲線近似回退區的效率變化；這是便於系統層分析的工程近似，不是完整的電路模型：

ξs,v​t,θ,θ3dB=min​ξmax, ξmaxps,v​t,θ,θ3dBpsat.(3.15a)

The square root follows from the ideal class-B approximation: with Vo the output voltage amplitude at fixed supply voltage and load, the RF output power satisfies PRF∝Vo2 while the average DC input satisfies PDC∝Vo, so ξ=PRF/PDC∝PRF. At one quarter of the saturated output the efficiency is ξmax/2, not ξmax/4. The values of ξmax and BO are listed in Section 5.1, a Ka-band satellite-downlink GaN Doherty MMIC reports a saturated power-added efficiency of 23–31%, and about 20% at 6 dB output back-off [24], which is of the same order as this model without being the same measurement point. Morello and Mignone’s two-carrier DVB-DSNG example uses 5.5 dB of output back-off per carrier [25], showing that multi-carrier operation needs a larger back-off to stay quasi-linear, that statement does not generalize to all multi-carrier systems and is cited here only as corroboration of the back-off magnitude.

平方根可由類 B 的理想近似理解：令 Vo 為輸出電壓振幅，在固定供應電壓與負載下射頻輸出功率滿足 PRF∝Vo2，平均直流輸入功率則近似滿足 PDC∝Vo，因此 ξ=PRF/PDC∝PRF。射頻輸出為飽和功率的四分之一時，效率是 ξmax/2 而不是 ξmax/4。ξmax 與 BO 的數值列於第 5.1 節；Ka 頻段衛星下行 GaN Doherty MMIC 的實測飽和功率附加效率為 23–31%，6 dB 輸出回退時約 20% [24]，與本模型的量級一致，但並非同一測量點。Morello 與 Mignone 的雙載波 DVB-DSNG 範例對每一載波採 5.5 dB 輸出回退 [25]，說明多載波操作為維持準線性區需要較大的回退；該敘述不足以泛化到所有多載波系統，此處僅作為回退量級的旁證。

This shape has one consequence that matters for this thesis: because efficiency is poorer at low output, lowering the transmit power does not lower the consumed power proportionally. What actually governs energy efficiency is how much throughput a given amount of power buys, and that is why Section 3.2 takes energy efficiency, rather than transmit power alone, as the first objective.

這個形狀有一個對本論文重要的後果：由於低輸出時效率較差，把發射功率壓低並不會等比例降低功率消耗。真正影響能量效率的是「用多少功率換到多少吞吐量」，這也是第 3.2 節把能量效率而非單純發射功率作為第一個目標的理由。

Let Pf(t) aggregate all fixed/circuit overhead that is not directly controlled by off-axis angle. With Nsact(t)=v∈V​zs,v(t) the number of active beams on satellite s, and with one active beam mapped to one RF chain in this study,

令 Pf(t) 彙總所有不直接由偏軸角控制的 fixed／circuit overhead。令 Nsact(t)=v∈V​zs,v(t) 為衛星 s 的啟用波束數；本研究將一支啟用波束對應到一條射頻鏈，則

Pf(t)=s∈S​Nsact(t) Pcir+1​Nsact(t)>0PBB.(3.16a)

The baseband power is shared by all active beams of that satellite and is therefore counted once per satellite, never twice. The values of Pcir and PBB are taken from Table II of You et al. [26] and are listed in Section 5.1. The one-beam-to-one-RF-chain mapping is this study’s own, not a conclusion about satellite architecture from that work, that work also includes local-oscillator and phase-shifter power, which this study does not adopt, so Eq. (3.16a) is a partial payload-power model rather than a complete satellite power model. The total system power is

基頻功率由該衛星的所有啟用波束共用，因此每顆衛星只計一次，不會重複計入。Pcir 與 PBB 的數值取自 You 等人 Table II [26]，列於第 5.1 節。「一支波束對應一條射頻鏈」是本研究的對應設定，不是該文的衛星架構結論；該文另含本振功率與相移器功率，本研究不納入，因此式 (3.16a) 是 partial payload-power model，不是完整的衛星總功率模型。系統總功率為

PN​t,θ,θ3dB=Pf(t)+s′∈S​v′∈V​zs′,v′(t) Ps′,v′p​t,θ,θ3dB.(3.16)

The sum runs over (s,v) rather than over (u,s,v): an active beam has a single amplifier, and its supply-side power is not counted once per user it carries, which is consistent with the ps,v aggregation of Eq. (3.12a). A per-link sum would add a spurious term s,v​(Us,v(t)−1)Ps,vp​t,θ,θ3dB that grows with occupancy and would make the first objective duplicate the work of the third.

此處對 (s,v) 求和而非對 (u,s,v) 求和：一支已啟用的波束只有一個放大器，其電源端功率不隨波束上載有幾位使用者而重複計入，這與式 (3.12a) 的 ps,v 聚合一致。若改為逐鏈路加總，多出的項為 s,v​(Us,v(t)−1)Ps,vp​t,θ,θ3dB，隨佔用人數單調上升，會使第一目標重複承擔第三目標的工作。

Finally, the displayed EE for a fixed link (u,s,v) is

最後，固定鏈路 (u,s,v) 的 EE 顯示量定義為

ηu,s,v​t,θu,s,v,θ3dB=Ru,s,v​t,θu,s,v,θ3dBPN​t,θ,θ3dB,  PN​t,θ,θ3dB>0.(3.17)

The three subscripts fix the numerator’s UE-link. The denominator remains shared system power, so ηu,s,v is a displayed single-link contribution to system EE, not true private-power per-user EE.

這裡的三下標固定分子的 UE-link；分母仍是共同系統功率，因此 ηu,s,v 是單一 UE-link 對系統 EE 的顯示量／貢獻，不宣稱為 private-power 的 true per-user EE。

The multi-objective deep Q-network (MODQN) baseline used in this study trains one Q-network for each of the three objectives, throughput was originally its first reward term [2]. This study defines the first reward as the sum of the selected-link EE contributions, so that transmission volume, beam angle, and power consumption are considered together.

本研究採用的多目標深度 Q 網路（MODQN）基準，以三個 Q 網路分別學習三個目標；其中吞吐量原本是第一個獎勵項 [2]。本文將第一個獎勵定義為所選鏈路 EE 貢獻的總和，使模型同時考慮傳輸量、波束角度與功率消耗。

## 3.2 Problem Formulation

This study considers energy efficiency, handover cost, and load balancing jointly. Let Lw be the number of visible satellites included at each step. Using the realized connection xu,s,v(t) as the decision outcome, the long-term objectives are [2]

本研究同時考慮能量效率、換手成本與負載平衡。令 Lw 為每個時間步納入決策的可見衛星數。以實際連線 xu,s,v(t) 為決策結果，長期目標可寫為 [2]

P1:& maxt​u∈U​r1,u​t,θ,θ3dB,P2:& mint​u∈U​Ψu(t),P3:& mint​s∈S​v∈V​Us,v(t)2,s.t.:& xu,s,v(t)∈{0,1},  s∈S​v∈V​xu,s,v(t)≤1.(3.24)

P1 seeks to deliver more data for the consumed power, P2 reduces handovers, and P3 drives the per-beam user counts toward an even split: with the served total fixed, the sum of squares is smallest when beam occupancies differ by at most one. The three objectives are defined below.

P1 希望在相同耗能下傳送更多資料，P2 減少換手，P3 則使各波束承載的使用者數趨於平均：在總服務人數固定時，平方和的最小值恰為各波束人數相差至多一人的配置。以下分別定義三個目標。

### 3.2.1 Angle-Aware Energy Efficiency

Equations (3.15)–(3.17) define link-side consumption, shared system power, and the displayed EE for a fixed link. The first reward sums the EE contributions of the links selected by each user:

式 (3.15)–(3.17) 已先定義鏈路耗電、共同系統功率與固定鏈路 EE 顯示量。第一個獎勵則把使用者實際選取的鏈路 EE 貢獻相加：

r1,u​t,θ,θ3dB&=s∈S​v∈V​xu,s,v(t)ηu,s,v​t,θu,s,v,θ3dB&=s∈S​v∈V​xu,s,v(t)Ru,s,v​t,θu,s,v,θ3dBPN​t,θ,θ3dB,  PN​t,θ,θ3dB>0.(3.25)

The unit of r1,u​t,θ,θ3dB is bit/J. It is the additive contribution of user u to instantaneous system energy efficiency. Equation (3.25) is defined when shared system power is positive, unselected links are excluded by xu,s,v(t)=0. No user-level SINR or user-level throughput symbol is introduced here.

r1,u​t,θ,θ3dB 的單位為 bit/J，表示使用者 u 對瞬時系統能量效率的可加貢獻。式 (3.25) 只在共同系統功率為正時成立；未選取的鏈路由 xu,s,v(t)=0 排除。這裡不另建立 user-level SINR 或 user-level throughput 符號。

The off-axis angle therefore enters GT directly, while the wanted-link numerator in Eq. (3.13) uses pu,s,vHu,s,vGT(θu,s,v,θ3dB), Eqs. (3.11)–(3.12) use the segment-start power p0 and the previous-step recurrence to determine angle-aware link RF power. The same p also enters the total-interference system state and the power/EE relations in Eqs. (3.15)–(3.17). The physical formula needs neither target-SINR inversion nor a protective power cap.

因此，角度直接進入 GT，而式 (3.13) 的 wanted-link numerator 使用 pu,s,vHu,s,vGT(θu,s,v,θ3dB)；式 (3.11)–(3.12) 以 segment-start p0 與 previous-step recurrence 決定角度相關鏈路 RF power。同一個 p 也進入總干擾所對應的系統狀態，以及式 (3.15)–(3.17) 的功率與 EE。主公式不需要目標 SINR、需求功率反推或保護性 cap。

### 3.2.2 Handover Cost

Using the serving pair (ρu(t),δu(t)) defined above, the cost of switching beams within one satellite is φ1, while the cost of switching satellites is φ2, with 0<φ1<φ2. The second reward is

沿用上述 (ρu(t),δu(t)) 表示的服務衛星與波束。同衛星換束的成本為 φ1，跨衛星換手的成本為 φ2，且 0<φ1<φ2。第二個獎勵為

r2,u(t)=−Ψu(t),  Ψu(t)=0,(ρu(t),δu(t))=(ρu(t−1),δu(t−1)),φ1,ρu(t)=ρu(t−1), δu(t)≠δu(t−1),φ2,ρu(t)≠ρu(t−1).(3.27)

Keeping the current connection has no cost, an intra-satellite beam switch has the smaller cost, and an inter-satellite handover has the larger cost.

因此，維持原連線時不產生成本，同衛星換束次之，跨衛星換手的成本最高。

### 3.2.3 Load Balancing

The third reward is the negative occupancy of the beam the user selected, where that occupancy is the Us,v(t) of Eq. (3.3):

第三個獎勵取使用者所選波束當步服務人數的負值；該人數即式 (3.3) 定義的 Us,v(t)：

r3,u(t)=−Ubu(t)(t).(3.28)

Here bu(t) is the beam user u selects at that step and Ubu(t)(t) is the number of users that beam serves at that step, from (3.28). A more crowded selected beam gives a smaller reward. This reward is decomposable per user: since u∈U​Ubu(t)(t)=s,v​Us,v(t)2, the sum over users is exactly the negative of the P3 objective, so each user’s reward is an exact decomposition of the global objective rather than one system-level scalar shared by every user.

其中 bu(t) 為使用者 u 當步所選的波束，Ubu(t)(t) 為該波束當步服務的使用者數（式 (3.3)）。所選波束越擁擠，獎勵越低。此獎勵逐使用者可分解：由 u∈U​Ubu(t)(t)=s,v​Us,v(t)2，所有使用者的獎勵總和恰為 P3 目標的負值，故每位使用者的獎勵是全域目標的精確分解，不同於舊式以系統層級純量共享給所有使用者的作法。

### 3.2.4 Reward Vector and Evaluation Metric

MODQN retains the three-dimensional reward vector [2]:

MODQN 保留三維獎勵向量 [2]：

Ru​t,θ,θ3dB=r1,u​t,θ,θ3dB, r2,u(t), r3,u(t).(3.29)

The three components do not take the same arguments: r1,u depends on the whole set of off-axis angles and beamwidth parameter through ηu,s,v and PN and therefore carries θ,θ3dB, whereas r2,u is fixed by the change of serving satellite and beam and r3,u by the occupancy alone, so neither involves an angle. The vector carries θ,θ3dB because its first component does. The generic rj,u and rj,u of Chapter 4 do not spell the arguments out component by component, each component keeps the definition given here.

三個分量的引數不同：r1,u 透過 ηu,s,v 與 PN 依賴整組偏軸角與波束寬參數，故帶 θ,θ3dB；r2,u 只由服務衛星與波束的變動決定、r3,u 只由佔用人數決定，兩者都不含角度。向量本身因第一個分量而帶 θ,θ3dB。第四章的通式 rj,u 與 rj,u 不逐項分寫引數，各分量沿用此處的定義。

Training uses stepwise rewards. Cross-time total-bits/total-energy evaluation belongs to the experimental procedure in Chapter 5, this section does not introduce a second physical EE symbol. Chapter 5 specifies the evaluation interval, service quality, coverage, and handover metrics separately from the per-step link EE in Eq. (3.17).

訓練使用逐步獎勵；跨時間的總位元／總能量評估屬於第五章的實驗評估程序，不在本節再建立第二個 EE 物理符號。第五章另行說明評估時間範圍、服務品質、覆蓋與換手指標，並將它們與式 (3.17) 的當步鏈路 EE 區分。

MODQN lets each user independently select the feasible beam with the largest weighted Q-value. Users with similar states may concentrate on the same few beams. By Eq. (3.14), users on one beam share its bandwidth, so concentration directly lowers each user’s rate and worsens the P3 objective in Eq. (3.24). Chapter 4 addresses this issue through two training-shaping strategies.

MODQN 讓每位使用者各自選取加權 Q 值最高的可行波束。狀態相近的使用者可能共同集中到少數波束；由式 (3.14)，同一波束上的使用者均分頻寬，集中會直接壓低每個人的速率，也使式 (3.24) 的 P3 目標惡化。第四章將以兩種訓練塑形策略處理這個問題。

# 4. Multi-Catfish Reinforcement Learning (MCRL)

## 4.1 Method Overview

Chapter 3 used the realized connection xu,s,v(t) to describe the physical system. This chapter organizes the selectable beams into a fixed action space for the neural network and explains how MODQN and MCRL use this information.

第三章以實際連線 xu,s,v(t) 描述物理系統。本章則要把可選波束整理成神經網路可處理的固定動作空間，並說明 MODQN 與 MCRL 如何使用這些資訊。

The physical beams available to a user change as the satellites move, but the output length of the neural network must remain fixed. Each user therefore uses a fixed-length candidate table. It retains Lw visible satellites and Jw candidate beams from each satellite, giving C=LwJw positions. Let C={1,…,C} be the candidate-index set, where c∈C denotes one table position.

低軌衛星持續移動，每位使用者可選的實際波束會改變，但神經網路的輸出長度必須固定。因此，每位使用者使用一張固定長度的候選表。表中保留 Lw 顆可見衛星，每顆衛星提供 Jw 個候選波束，共有 C=LwJw 個位置。令 C={1,…,C} 為候選編號集合，c∈C 表示表中的一個位置。

The table contents change over time. The mapping bu(c,t)∈S×V gives the physical satellite and beam represented by position c for user u at time t. The network selects a candidate index, and bu(c,t) maps that choice back to the physical indices used in Chapter 3.

候選表的內容會隨時間更新。bu(c,t)∈S×V 表示候選位置 c 在時間 t 對使用者 u 所對應的實際衛星與波束。後續網路選的是候選編號，實際連線則透過 bu(c,t) 回到第三章的衛星－波束索引。

This candidate table is a thesis-side fixed-output representation. The original MODQN paper writes its state and access action over the full L×V beam-position grid, this thesis restricts each moving visibility window to Lw×Jw candidates.

這張候選表是本論文為固定網路輸出所建立的表示法。原始 MODQN 論文把狀態與 access action 寫在完整的 L×V 波束位置網格上；本論文則將移動中的可見視窗限制為 Lw×Jw 個候選。

The user state contains four types of information: the previous connection, a link-level signal-quality value for each candidate, the off-axis angle of each candidate, and the number of users who selected each beam at the previous step. In candidate-table order, the state is

使用者的狀態包含四類資訊：前一步的連線、各候選鏈路的訊號品質、各候選的偏軸角，以及前一步選向各波束的使用者數。將它們依候選表順序排列後，狀態寫成

su(t)&=concat​xu(t−1), (γu,s,v​t,θu,s,v,θ3dB)(s,v)=bu(c,t)c∈C, (θu,s,v(t))(s,v)=bu(c,t)c∈C, Nu(t−1).(4.1)

Here, xu(t−1)=(xu,bu(c,t)(t−1))c∈C records the previous connection. The two parenthesized sequences list the three-index link SINR and off-axis-angle elements in candidate order, and concat joins the four blocks into one state vector. The SINR reuses the single physical definition of γu,s,v from Chapter 3. These values are available to the environment before the current action. If the implementation uses cached or predicted provenance, that label belongs in field metadata and does not create another physical symbol. The candidate table at step t must be ordered so that the beam that actually served the user at the previous step remains identifiable: if user u was served at t−1, its serving beam (ρu(t−1),δu(t−1)) must still occupy one candidate position, so that xu(t−1) recovers the previous physical association required by Eq. (3.27), if that beam has left the candidate table, every position selected at this step constitutes a handover.

其中，xu(t−1)=(xu,bu(c,t)(t−1))c∈C 記錄前一步的連線。兩個圓括號序列依候選順序排列三下標的鏈路 SINR 與偏軸角元素，concat 再把四個區塊接成一個狀態向量；SINR 沿用第三章唯一的 γu,s,v 定義。這些值在動作前由環境可取得；若實作使用 cached 或 predicted provenance，該標記放在資料欄位 metadata，不改變公式中的物理符號。候選表在時間 t 的排列必須讓上一步實際服務的波束仍可辨識：若使用者 u 在 t−1 被服務，其服務波束 (ρu(t−1),δu(t−1)) 必須仍占有某一個候選位置，xu(t−1) 才能還原出式 (3.27) 所需的前一步實體關聯；若該波束已離開候選表，則本步無論選哪一格都構成換手。

Let ns,v(t−1) denote the demand directed to beam (s,v) at the previous step. The vector Nu(t−1)=[nbu(c,t)(t−1)]c∈C follows the candidate order, whereas Us,v(t) in Eq. (3.3) is the realized served load.

令 ns,v(t−1) 表示前一步選向波束 (s,v) 的使用需求量。Nu(t−1)=[nbu(c,t)(t−1)]c∈C 是依候選順序排列的向量；式 (3.3) 的 Us,v(t) 則表示實際服務負載。

Equation (4.1) uses only information available before the current action is selected. The connection, load, SINR, and throughput produced by that action determine the current reward and the next state.

式 (4.1) 只使用選擇動作前可取得的資訊；本步動作造成的連線、負載、SINR 與吞吐量則用於產生本步獎勵及下一步狀態。

The candidate-table length is fixed, but some positions may be unavailable. Let mu(t) denote the feasible-action mask:

候選表的長度固定，但部分位置在當下可能不可用。以 mu(t) 表示可行動作遮罩：

mu(t)=mu,1(t),mu,2(t),…,mu,C(t),  mu,c(t)∈{0,1},(4.2)

Au(t)=c∈C∣mu,c(t)=1.(4.3)

Here, mu,c(t)=1 means that candidate c is selectable and 0 means that it is not. The set Au(t) collects all selectable candidate indices. The policy returns the scalar candidate index au(t), the environment uses its one-hot encoding au,c(t) when it applies the action to candidate c.

其中，mu,c(t)=1 表示候選 c 可選，0 表示不可選；Au(t) 收集所有可選的候選編號。政策輸出的是純量候選編號 au(t)；環境套用動作時，才使用它在候選 c 上的 one-hot 編碼 au,c(t)。

Let the scalar au(t) denote the candidate selected by user u. The environment expands it into the one-hot indicator au,c(t), which is 1 only for the selected candidate:

使用者 u 選出的候選編號記為純量 au(t)。為了讓環境逐一套用到候選位置，將它展開為 one-hot 指示量 au,c(t)；候選 c 被選中時該分量才為 1：

au(t)∈Au(t),  au,c(t)=1​c=au(t), c∈C.(4.4)

When the feasible-action set is non-empty, each user selects one candidate at each time step, when it is empty, no candidate can be selected, so

可行動作集合非空時，每位使用者在一個時間步選擇一個候選；可行動作集合為空時沒有任何候選可選，因此

c∈C​au,c(t)=1,Au(t)≠⌀,0,Au(t)=⌀,  au,c(t)∈{0,1}.(4.5)

When Au(t)=⌀, the condition au(t)∈Au(t) of Eq. (4.4) has no solution and no candidate is executed at that step: user u is unserved, all its xu,s,v(t) are zero, and the transition of that step does not enter the replay buffer. This setting does not claim that the mask is always non-empty, because candidate-cell coverage and link feasibility can together rule out every candidate at the same step.

Au(t)=⌀ 時式 (4.4) 的 au(t)∈Au(t) 無解，該步不執行任何候選：使用者 u 於該步 unserved，其所有 xu,s,v(t) 為 0，該步的轉移不進入經驗回放。本設定不宣稱遮罩恆為非空，因為候選格覆蓋與鏈路可行性可能在同一步同時排除所有候選。

The decision-time mask mu,c(t) and the physical-beam activation zs,v(t) have different roles: the first determines what a user may select, while the second follows the activation rule of Eq. (3.4) and states whether that physical beam radiates. The realized connection is therefore

決策時的遮罩 mu,c(t) 與實體波束啟用變數 zs,v(t) 的功能不同：前者決定使用者能選什麼，後者由式 (3.4) 的啟用規則決定該實體波束是否輻射。實際連線為

xu,bu(c,t)(t)=au,c(t) zbu(c,t)(t).(4.5a)

Thus, a user is connected only when the candidate was selected and the corresponding beam radiates. The link must then also be feasible: a link whose required power exceeds the per-beam limit is declared infeasible, and that user is in outage for the step. The selections of all users form the joint action:

因此，候選必須被選中且對應波束確實在輻射，使用者才會連上該波束。連上之後仍須滿足鏈路可行性：所需功率超過每波束上限者判為不可行，該使用者於該步為 outage。所有使用者在同一時間步的選擇合稱為聯合動作：

a(t)=a1(t),a2(t),…,aU(t).(4.6)

Valid selections determine beam activation, whose state follows Eq. (3.4). Equation (4.5a) then gives the realized connections, from which the next state and the three rewards are calculated.

各波束依有效選擇形成啟用結果，其啟用狀態由式 (3.4) 決定。接著由式 (4.5a) 得到實際連線，再計算下一個狀態與三個獎勵。

Together, the state, action, reward vector, environment transition, and discount factor form a multi-objective Markov decision process.

以上狀態、動作、獎勵向量、環境轉移與折扣因子共同構成多目標馬可夫決策過程。

The original MODQN builds one Q-network for each objective, giving Q(su,a)=[Q1(su,a),Q2(su,a),Q3(su,a)] [2]. The three Q-values are combined with weights Ω=[ω1,ω2,ω3], and each user selects the action with the largest combined value from Au(t), ϵ-greedy exploration is added during training. Each Q-network is updated by temporal-difference learning using its own objective reward. The concrete weights are reported in Section 5.1.

原始 MODQN 為三個目標各建立一個 Q 網路，得到 Q(su,a)=[Q1(su,a),Q2(su,a),Q3(su,a)] [2]。三個 Q 值以權重 Ω=[ω1,ω2,ω3] 加權後，每位使用者從 Au(t) 中選出綜合價值最高的動作；訓練期間再加入 ϵ-greedy 探索。各 Q 網路使用自己的目標獎勵進行時間差分更新。權重的具體設定列於第 5.1 節。

MCRL retains MODQN’s three objective Q-networks and per-user independent argmax selection. Experience shaping and reward shaping act only during training, modifying the training experiences and the catfish agent’s first reward respectively, the input representation and the selection rule at deployment are the same as in MODQN.

MCRL 沿用 MODQN 的三個目標 Q 網路與逐使用者 argmax 選擇。經驗塑形與獎勵塑形只在訓練期間作用，分別調整訓練經驗與鯰魚代理的第一個獎勵；部署時的輸入表示與選法都與 MODQN 相同。

Figure 4-1 locates these mechanisms in data collection and parameter updating.

圖 4-1 標示各機制在資料蒐集與參數更新中的作用位置。

Fig. 4-1 Intervention points in the MODQN update

Fig. 4-1: Intervention points of MCRL in the MODQN update. Energy-efficiency stratification and competitive reward process catfish experiences, periodic intervention supplies a mixed batch for the main agent, asymmetric discounting distinguishes the two within-episode weightings.

Figure 4-2 presents the correspondence between the main and catfish agents at the objective-network level.

圖 4-2 以目標網路為單位呈現主代理與鯰魚代理的對應關係。

Fig. 4-2 Objective-network view of MCRL

Fig. 4-2: Objective-network view of MCRL. The main and catfish sides each contain three objective Q-networks, with the one-to-one correspondence QjM↔QjF↔rj. Each side scalarizes its three Q-values using the common weights Ω and selects one action.

Figure 4-3 shows the trajectories, replay buffers, and periodic intervention from the agent-and-data-flow perspective.

圖 4-3 補充兩個代理的軌跡、經驗池與週期性介入關係。

Fig. 4-3 Role-level MCRL training architecture

Fig. 4-3: Agent-and-data-flow view of MCRL. The main and catfish agents each generate a training trajectory and maintain a replay buffer. Catfish transitions are routed by their first-objective score, and periodic intervention uses a mixed batch to update the main agent, only the main agent is deployed after training.

## 4.3 Experience Shaping with the Catfish Agent

This section explains how MCRL uses an auxiliary catfish role in the three-objective handover problem.

本節說明 MCRL 如何在三目標換手問題中使用鯰魚輔助角色。

The main agent and the catfish agent are each a three-objective MODQN. Both sides contain three online Q-networks and three target networks. QjM and QjF correspond to the same objective rj, where j=1,2,3 denote energy efficiency, handover, and load balancing. Within each side, the three objective networks follow the deep Q-network training procedure [27] and use the same states, actions, transitions, and replay buffer, while each reads the jth component of the reward vector. Training therefore has two MODQN agents, after training, only the main agent is retained.

主代理和鯰魚代理各是一組三目標 MODQN。兩端都使用三個線上 Q 網路和三個目標網路，QjM 與 QjF 分別對應同一個目標 rj，其中 j=1,2,3 依序代表能量效率、換手和負載平衡。兩端各自學習自己的軌跡和經驗；每一端的三個目標網路沿用深度 Q 網路的訓練流程 [27]，共用該端的狀態、動作、轉移與經驗池，再讀取獎勵向量中各自的第 j 個分量。訓練時共有兩組 MODQN；完成訓練後只保留主代理。

The three rewards have different units and scales. After the environment produces a reward and before it enters a replay buffer, component j is divided by a fixed positive constant cj, so that rj,ud=rj,ud/cj, the complete update expression is given in Eq. (4.13). The term “unshaped reward” denotes this calibrated reward before the competitive term in Eq. (4.11) is added.

三個獎勵的單位和量級不同。環境產生獎勵後、寫入經驗池前，第 j 個分量先除以固定正數 cj，亦即 rj,ud=rj,ud/cj；完整更新式列於式 (4.13)。本文將未加入式 (4.11) 競爭項、但已完成尺度校準的獎勵稱為「未整形獎勵」。

The main-side buffer DM stores the unshaped, calibrated three-dimensional reward. The catfish-side buffer DF retains both unshaped and competitively shaped calibrated views: catfish TD updates use the shaped view, whereas catfish samples used by periodic intervention use the unshaped view. Only the energy-efficiency stratification threshold directly uses the first objective before calibration. Both buffers operate on complete time-step transitions containing all users.

主代理經驗池 DM 儲存未整形且已校準的三維獎勵。鯰魚經驗池 DF 同時保留未整形與競爭整形後的校準視圖：鯰魚代理 TD 更新使用整形視圖，週期性介入抽出的鯰魚樣本則使用未整形視圖。只有能效分層的門檻直接使用校準前的第一目標。兩個經驗池都以包含全體使用者的完整時間步轉移為單位。

Experience shaping has three components. Energy-efficiency stratification selects catfish experiences, asymmetric discounting changes how the catfish agent weights returns across steps within an episode, and periodic intervention uses part of the catfish experience for an additional main-agent update.

經驗塑形由三個部分組成：能效分層負責篩選鯰魚經驗，非對稱折扣改變鯰魚代理在回合內對各步回報的加權，週期性介入則把部分鯰魚經驗用於主代理的額外更新。

The first component is energy-efficiency stratification. At each step, the catfish side produces one complete transition bundle containing all users and the three-dimensional reward, denoted τtF. Define its stratification score as the mean unshaped additive contribution to the first objective before calibration: gF​t,θ,θ3dB=U−1u∈U​r1,uF​t,θ,θ3dB. This score is then compared with the empirical distribution FW of the scores from the most recent W bundles. It is used only for training-time stratification and is not an evaluation metric. For q∈(0,1), FW−1(q) is the q-quantile of the empirical distribution, meaning that approximately a proportion q of recent scores are no greater than this threshold. Let q1 and q2 satisfy 0<q1<q2<1. The routing rule is:

第一個部分是能效分層。鯰魚代理在每一步產生一筆包含全體使用者和三維獎勵的完整轉移束 τtF。令分層分數為該步校準前、未整形第一目標的平均可加貢獻： gF​t,θ,θ3dB=U−1u∈U​r1,uF​t,θ,θ3dB。 此分數再和最近 W 束分數的經驗分布 FW 比較。此分數僅用於訓練分層，不作為評估指標。對 q∈(0,1)，FW−1(q) 是經驗分布的 q 分位數，表示約有比例 q 的近期分數不高於該門檻。令 q1 和 q2 滿足 0<q1<q2<1，分流規則如下：

routing​τtF=DF,gF​t,θ,θ3dB≥FW−1(q2),DM,FW−1(q1)≤gF​t,θ,θ3dB<FW−1(q2),⌀,gF​t,θ,θ3dB<FW−1(q1).(4.7)

High-scoring bundles enter DF, middle-scoring bundles are also sent to DM, and low-scoring bundles are discarded. Although routing uses only the first objective before calibration, the retained item is the complete three-objective transition, the replay buffer then uses the calibrated view described above.

高分轉移束放入 DF，中間分數的轉移束另外送入 DM，低分轉移束則捨棄。分流雖然只看校準前的第一個目標，保留的仍是完整三目標轉移；寫入經驗池的獎勵則使用前述校準後的視圖。

Fig. 4-4 Energy-efficiency stratification

Fig. 4-4: Energy-efficiency stratification. The catfish-side transition bundle τtF is scored by its mean unshaped first-objective contribution gF​t,θ,θ3dB before calibration and compared with the recent distribution FW. High-scoring bundles enter DF, middle-scoring bundles are also sent to DM, and low-scoring bundles are discarded (Eq. (4.7)), each retained buffer then uses its corresponding calibrated reward view.

The second component is asymmetric discounting. The three main-side objectives share βM, while the three catfish-side objectives share the larger βF:

第二個部分是非對稱折扣。主代理的三個目標共用 βM，鯰魚代理的三個目標共用較大的 βF：

βM<βF,  βjM≡βM, βjF≡βF, j=1,2,3.(4.8)

A larger discount factor keeps more of the return from later steps within an episode. At the episode length H=10 used here, both effective horizons are at least the episode itself (1/(1−βM)=10), so the difference appears as how weight is distributed across steps within an episode: the main side emphasizes nearer-term returns, while the catfish side counts the whole episode almost uniformly. The episode length matches the effective horizon of βM at ten steps, which is the setting of Table I in the source paper and therefore not a deviation.

折扣因子越大，回合內較後步驟的回報保留得越多。在本文的回合長度 H=10 下，兩端的有效視野皆不短於回合本身（1/(1−βM)=10），故差異體現為回合內各步權重的分配：主代理較重視近期回報，鯰魚代理則接近均勻計入整個回合。回合長度與 βM 的有效視野一致，均為 10 步，此為原論文 Table I 的設定，不構成偏離。

Fig. 4-5 Asymmetric discounting

Fig. 4-5: Asymmetric discounting. The three main-side objective networks use βM, and the three catfish-side objective networks use the larger βF (Eq. (4.8)). The relative weight of a return n steps ahead is βn, the bars only illustrate this decay.

The third component is periodic intervention. Each interval T is sampled uniformly from the integer range [T1,T2]. When the interval expires, a mixed batch BI is formed from a proportion ρI of DF and the remainder from DM:

第三個部分是週期性介入。每次從整數區間 [T1,T2] 均勻抽取介入間隔 T。間隔到期時，從 DF 取出比例 ρI 的樣本，和 DM 的樣本合成一個混合批次 BI：

nF&=round​ρIB,nM&=B−nF.(4.9)

Here, B is the batch size and round(⋅) denotes rounding to the nearest integer. Drawing nM samples from DM and nF samples from DF forms the mixed batch BI, both kinds of samples use the unshaped, calibrated three-dimensional reward. The three main-side networks use the same BI and perform one additional update with βM, a new T is drawn after an intervention.

其中 B 是批次大小，round(⋅) 表示取最近整數。由 DM 抽取 nM 筆樣本、由 DF 抽取 nF 筆樣本，合併成混合批次 BI；兩類樣本都使用未整形且已校準的三維獎勵。主代理的三個目標網路共同使用 BI，並以 βM 額外更新一次；介入後重新抽取 T。

Fig. 4-6 Periodic intervention

Fig. 4-6: Periodic intervention. nM main-side samples and nF catfish-side samples form BI (Eq. (4.9)), both use unshaped, calibrated rewards. The same batch then updates Q1M, Q2M, and Q3M.

Together, the three components perform experience shaping, and they change only training, deployment still selects actions with the main agent.

三個部分共同完成經驗塑形，且都只改變訓練；部署時仍由主代理選擇動作。

The two sides use the same action-selection rule. Let d∈{M,F} denote the main or catfish side, and let Aud(t) be the feasible action set for user u on that side. Each side combines its three Q-values with the common weights Ω=[ω1,ω2,ω3] from Section 3.2 and selects from this set. During training, each side also uses its own ϵ-greedy exploration:

兩端的選動作方式相同。令 d∈{M,F} 表示主代理端或鯰魚代理端，Aud(t) 表示使用者 u 在該端的可行動作集合。兩端都用第 3.2 節的共同權重 Ω=[ω1,ω2,ω3] 合成三個 Q 值，再從這個集合中選動作；訓練時另外加入各端自己的 ϵ-greedy 探索：

Vud(a,t)=j=13ωjQjdsud(t),a,  aud(t)=argmaxa∈Aud(t)Vud(a,t), d∈{M,F}.(4.10)

Equation (4.13) shows the one-to-one correspondence between the two MODQNs. QjM and QjF estimate the same objective, and both sides combine their three objective values into the scalarized value Vud(a,t). Because the two sides learn independently, they may select different actions. The three networks within one side share its selected action but keep separate network parameters. These Q-networks are trained with calibrated rewards, so the effective trade-off weight in the original units is ωj/cj.

式 (4.10) 表示兩組 MODQN 的一一對應關係。QjM 和 QjF 估計同一個目標，兩端都把三個目標值合成一個綜合價值 Vud(a,t)。因為兩端獨立學習，所以可能選出不同動作。每一端的三個網路共用該端選出的動作，但各自保留網路參數。這些 Q 網路以校準後的獎勵訓練，因此原始單位下的實際取捨權重為 ωj/cj。

## 4.4 Reward Shaping: the Competitive Reward Mechanism

This section defines the first reward used by the catfish side. The competitive reward mechanism (ACRM) compares the catfish and main agents on the energy-efficiency objective. Handover and load balancing receive no competitive term, but they are still calibrated by the same cj before entering a replay buffer.

本節定義鯰魚代理使用的第一個獎勵。競爭獎勵機制（ACRM）比較鯰魚代理和主代理在能量效率目標上的表現。換手和負載平衡不加入競爭項，但寫入經驗池前仍依相同的 cj 校準。

ACRM compares two complete joint actions under the same pre-action state and stochastic conditions. The catfish agent uses Eq. (4.10) to produce aF(t), while the main agent produces the exploration-free greedy action aM∣F(t). The two simulations therefore differ only in their joint action. Their first objectives are both computed by Eq. (3.25). Let the raw rewards be r1,uF​t,θ,θ3dB and r1,uM∣F​t,θ,θ3dB, after which the difference and shaped reward are defined as:

ACRM 在相同的動作前狀態和隨機條件下比較兩個完整聯合動作。鯰魚代理依式 (4.10) 產生 aF(t)，主代理則產生不含探索的貪婪動作 aM∣F(t)。兩次模擬只有聯合動作不同，第一目標都依式 (3.25) 計算。令原始獎勵為 r1,uF​t,θ,θ3dB 和 r1,uM∣F​t,θ,θ3dB，接著定義競爭差值和整形獎勵：

r1,uS​t,θ,θ3dB=r1,uF​t,θ,θ3dB−r1,uM∣F​t,θ,θ3dB,  r1,uC​t,θ,θ3dB=r1,uF​t,θ,θ3dB+ηw r1,uS​t,θ,θ3dB.(4.11)

Here, ηw≥0 is the competitive weight. Both raw r1 values are calculated by Eq. (3.25) before the difference is formed. r1,uS>0 means that the catfish rollout has higher energy efficiency under the same conditions, whereas r1,uS<0 means the opposite. When ηw=0, r1,uC=r1,uF.

其中 ηw≥0 是競爭權重。兩個原始 r1 都先依式 (3.25) 計算，再形成差值。r1,uS>0 表示鯰魚代理在相同條件下的能效較高，r1,uS<0 則相反。當 ηw=0 時，r1,uC=r1,uF。

The catfish-side shaped vector is [r1,uC,r2,uF,r3,uF]. ACRM first forms r1,uC in the original units and then calibrates the entire vector. Energy-efficiency stratification still scores the uncalibrated r1,uF, samples sent to DM and the intervention batch BI use the unshaped calibrated view, catfish TD updates use the shaped calibrated view. The competitive term therefore affects only Q1F.

鯰魚代理的整形向量為 [r1,uC,r2,uF,r3,uF]。ACRM 先在原始單位下形成 r1,uC，再校準整個向量。能效分層仍以校準前的 r1,uF 評分；送入 DM 的樣本與介入批次 BI 使用未整形的校準視圖；鯰魚代理 TD 更新則使用整形後的校準視圖。因此，競爭項只影響 Q1F。

Fig. 4-7 Competitive reward mechanism

Fig. 4-7: Competitive reward on the energy-efficiency axis. Under the same pre-action state and stochastic conditions, only the complete joint action changes. Both simulations compute the first objective with Eq. (3.25), then form r1,uS and r1,uC (Eq. (4.11)). Both shaped and unshaped vectors are then calibrated by Eq. (4.13), and only Q1F uses the competitively shaped value.

## 4.5 Overall Training Procedure

The main and catfish agents generate separate training trajectories. The competitive reward compares the two sides using the main-agent value before the update, and experience routed at that step is available to later parameter updates.

主代理與鯰魚代理各自產生訓練軌跡。競爭獎勵使用更新前的主代理價值進行配對比較；當步完成分流的經驗則供後續參數更新使用。

Pseudocode 4-1 summarizes the complete order. E and H are the number of training episodes and the maximum steps per episode, B is the TD batch size, and K is the target-network synchronization period. The interval T is sampled from the range specified for Eq. (4.9).

偽程式碼 4-1 整理完整訓練順序。E 與 H 分別為訓練回合數與每回合最大時間步，B 為 TD 批次大小，K 為目標網路同步週期。介入間隔 T 依式 (4.9) 所述範圍抽取。

| # | Pseudocode 4-1: MCRL Training |
| --- | --- |
|  | Input: Two MODQNs and the training settings required by Eqs. (4.7)–(4.16). Output: the trained main agent QM. |
| 1 | Initialize the online and target networks, replay buffers DM,DF, and the first intervention interval T. |
| 2 | for e=1,…,E do: initialize separate main-agent and catfish-agent training trajectories. |
| 3 | &nbsp,&nbsp,for t=1,…,H do: construct the two inputs using Eqs. (4.7)–(4.9). |
| 4 | &nbsp,&nbsp,&nbsp,&nbsp,Select the two joint actions independently using Eq. (4.10) and ϵ-greedy exploration. |
| 5 | &nbsp,&nbsp,&nbsp,&nbsp,Execute the main-agent action to obtain τtM, calibrate its unshaped reward, and store the complete transition in DM. |
| 6 | &nbsp,&nbsp,&nbsp,&nbsp,Execute the catfish-agent action to obtain τtF, evaluate the main-agent greedy action from the same pre-action state and form the competitive reward using Eq. (4.11). |
| 7 | &nbsp,&nbsp,&nbsp,&nbsp,Route τtF using its uncalibrated first objective in Eq. (4.7). Samples sent to DM use the unshaped calibrated view, catfish TD updates use the shaped calibrated view. |
| 8 | &nbsp,&nbsp,&nbsp,&nbsp,When enough experience is available, update the main and catfish agents using βM and βF, respectively. |
| 9 | &nbsp,&nbsp,&nbsp,&nbsp,When the intervention interval expires, form BI using Eq. (4.9), apply one additional main-agent update, and sample a new T. |
| 10 | &nbsp,&nbsp,&nbsp,&nbsp,If the episode terminates, leave the time-step loop. |
| 11 | &nbsp,&nbsp,Synchronize both target networks every K episodes. |
| 12 | return QM. |

The main-side TD update reads the unshaped calibrated vector. The catfish-side TD update reads the calibrated form of [r1,uC,r2,uF,r3,uF]. Let d∈{M,F} denote the main or catfish side, and let βd be its discount factor. Both sides keep the form of Eq. (16) of the source paper, with each objective taking its own maximum over its own target network:

主代理的 TD 更新讀取未整形的校準向量；鯰魚代理的 TD 更新讀取 [r1,uC,r2,uF,r3,uF] 校準後的向量。在此令 d∈{M,F} 表示主代理或鯰魚代理，並以 βd 表示該端的折扣因子。兩端沿用原論文式 (16) 的形式，每個目標各自在自己的目標網路上取最大值：

yj,ud=rj,ud(t),sud(t+1) is a terminal state,rj,ud(t)+βdmaxa′∈Aud(t+1)Qjdsud(t+1),a′,θjd,−,otherwise,  d∈{M,F}, j=1,2,3.(4.12)

If the next state is terminal the TD target is the step reward alone, otherwise the discounted target-network value is added, and each objective takes its own maximum over its own target network. The main and catfish sides each perform one TD update, periodic intervention performs one additional main-side update with BI.

若下一個狀態是終止狀態，TD target 只有當步獎勵；否則再加上折扣後的目標網路價值，且每個目標各自在自己的目標網路上取最大值。主代理與鯰魚代理各自做一次 TD 更新；週期性介入則用 BI 對主代理額外做一次 TD 更新。

Taking a separate maximum per objective during training is distinct from the deployment rule of (4.13), which scalarizes with Ω before the argmax. This is the design of the source paper and is retained unchanged.

訓練端每個目標各自取最大值，與部署端式 (4.10) 以 Ω 純量化後取 argmax 是兩件不同的事；這是原論文的設計,本文沿用,不另行更動。

The three rewards have different units and magnitudes: bit/J, handover cost, and a beam-occupancy count. The following calibration is a thesis-defined training contract, not a formula stated in the original MODQN paper. The original paper stores the raw reward components and directly scalarizes the learned Q-values with Ω, it does not introduce cj. This thesis adds cj so that the three objectives enter training on fixed, declared scales. Before a sample enters a replay buffer, each reward is divided by the fixed calibration constant cj:

三個獎勵的單位和量級不同：第一個的單位為 bit/J，第二個是換手成本，第三個是波束服務人數的計數。下列校準是本論文自己定義的訓練契約，不是原始 MODQN 論文明確寫出的公式。原論文保存原始 reward components，再直接以 Ω 對學到的 Q 值做純量化，沒有引入 cj。本論文為了讓三個目標以固定且明確宣告的尺度進入訓練，加入 cj；因此樣本放入經驗池前先除以固定的校準常數 cj：

rj,ud(t)=rj,ud(t)cj,  cj>0,  j∈{1,2,3},(4.13)

Here, rj,ud(t) is the jth update reward on side d before calibration and rj,ud(t) is the calibrated value. Before training starts, the constants cj are fixed and then shared by the training procedures. Consequently, the scalarization in Eq. (4.10) and the TD target in Eq. (4.12) use the same scale. In the original units, the effective trade-off weight is ωj/cj.

其中 rj,ud(t) 是端 d 校準前的第 j 個更新獎勵，rj,ud(t) 是校準後的值。訓練開始前固定 cj，並由各訓練程序共用。這樣式 (4.10) 的純量化和式 (4.12) 的 TD target 都使用同一尺度；原始單位下的實際取捨權重為 ωj/cj。

Deployment retains the three main-agent objective networks. Each user computes the scalarized value in Eq. (4.10) and uses argmax to select a service beam.

部署時只保留主代理的三個目標網路。每位使用者依式 (4.10) 計算綜合價值，並以 argmax 選出服務波束。

# 5. Experimental Results

## 5.1 Simulation Settings

This chapter specifies the simulation environment, index domains, channel and energy-consumption model, and the settings used for training and evaluation. Chapter 3 defines the single active EE contract; this chapter supplies numerical scenarios and evaluation scope without redefining SINR, throughput, or EE.

本章說明模擬環境、索引域、通道與能耗模型，以及訓練和評估設定。第三章的 active EE 主鏈是唯一的公式契約；本章只補充數值情境與實驗評估範圍，不重新定義 SINR、throughput 或 EE。

Table 5-1 gives the numerical settings of the environment and index domains. The symbol v in Chapter 3 denotes one beam index, while V denotes the number of beam positions on each satellite; this study sets V=39. The symbols U, S, V, and C correspond to the element counts of the respective sets.

表 5-1 列出環境與索引域的數值設定。第三章的 v 表示一個波束索引，V 表示每顆衛星的波束位置數；本研究設定 V=39。表中的 U、S、V 與 C 分別對應各集合的元素數。

Table 5-1. Environment and index-domain settings

表 5-1：環境與索引域設定

| Category | Symbol or relation | Setting in this study |
| --- | --- | --- |
| Satellite constellation | S​ | Real Starlink ephemerides (TLE with SGP4), 373 daily files covering 2025-07-27 to 2026-08-20; inclination about 53 degrees |
| Satellite altitude | hs​ | Determined by the ephemerides rather than set. The daily median altitude over the corpus is 483.0 km, and those daily medians range from 470.7 to 539.7 km; the instantaneous altitudes of individual satellites spread wider than that range |
| Visible-satellite window | Lw​ | 4 |
| Ground users | U​ | 100 |
| Beam-pointing locations per satellite | V​ | 39. Coverage is measured directly on the cell lattice at hs=483 km, where 39 cells cover 95.2% of the service area. The lattice takes Rb as the circumradius of the hexagonal cell, so the centre spacing is 3Rb=24.24 km and one cell covers 509.0 km​2; each cell is then inscribed in its own 3 dB contour and no coverage hole is left. The same area ratio gives 18000×0.95/509.0=33.6 cells, below the measured value, because it ignores the edge spill when the lattice is clipped to the rectangular service area; the measurement governs |
| Local candidate ranks per window satellite | Jw​ | 7 (the beam location nearest the user and its six adjacent locations) |
| Candidate action domain | C=LwJw​ | 4×7=28​ |
| Pointing combinations in the window | LwV​ | 4×39=156 satellite–beam-location pairs |
| Time discretization | Δt, H | 30.08 s per step and 10 steps per episode |

The two numbers refer to different model objects: 156 possible satellite–beam-location pairs in the current window, and 28 candidate actions available to each user at decision time.

表中的兩個數字分別代表不同的模型對象：156 是視窗內可指向的衛星－波束位置總數，28 是每位使用者可選的候選動作數。

With C=28, the four components in Eq. (4.1)—connection, candidate SINR, off-axis angle, and previous demand—each contain 28 entries. The original state su therefore has dimension 4C=112. That is also the dimension of the Q-network input.

在 C=28 下，式 (4.1) 的四類原始分量——前一步連線、候選 SINR、偏軸角與前一步需求——各有 28 個元素，因此 Q 網路輸入 su 為 4C=112 維。

Table 5-2 lists the experimental calibration values for the channel and energy model in Chapter 3. The main chain uses GT​θ,θ3dB, the linear link-power factor that does not directly carry the wanted-link angular pattern Hu,s,v(t), angle-aware pu,s,v, the per-beam Ps,vp, Pf, and PN; the detailed channel settings in the table belong only to scenario calibration and do not expand the main formula.

表 5-2 列出第三章通道與能耗模型的實驗校準值。正文主鏈使用 GT​θ,θ3dB、不直接承載 wanted-link 角度型樣的線性鏈路功率因子 Hu,s,v(t)、角度相關 pu,s,v、逐波束的 Ps,vp、Pf 與 PN；表中的細部通道設定只用於情境校準，不會擴張主公式。

Table 5-2. Channel and energy-model settings

表 5-2：通道與能耗模型設定

| Category | Symbol or relation | Setting in this study |
| --- | --- | --- |
| Carrier and bandwidth | fc, Bsys | 20 GHz and 500 MHz |
| Gaseous atmospheric absorption | Lg(α)=Azen/sinα​ | Azen=0.25 dB. The form is Eq. (6.6-8) of TR 38.811 [17]; the zenith value is recovered from the LEO Ka-band 20 GHz downlink link budget of TR 38.821 [22], whose atmospheric loss is 0.5 dB at the LEO target elevation of 30 degrees |
| Scintillation | Lc(α)​ | Tropospheric scintillation at 20 GHz from Table 6.6.6.2.1-1 of TR 38.811 [17]: 0.12 dB at 90 degrees, 0.30 dB at 30 degrees and 1.08 dB at 10 degrees of elevation. Section 6.6.6.1 of the same report considers ionospheric scintillation only below 6 GHz, so it is not counted at 20 GHz |
| Shadow fading | Ls(α)​ | Ka-band LOS shadow fading from Table 6.6.2-3 of TR 38.811 [17], zero-mean Gaussian in dB, with a standard deviation from 0.4 dB at 90 degrees to 1.9 dB at 10 degrees of elevation. Clutter loss applies only to NLOS and is not counted for a fixed LOS terminal |
| Frequency reuse | cs,v, Bw | Three-color reuse and 166.667 MHz |
| Beamwidth and beam coverage radius | θ3dB, Rb=hstan(θ3dB/2) | 3.32 degrees and 13.998 km at hs=483 km |
| Boresight gain | G0​ | 2000 in linear scale, or 33.010 dBi |
| Receive-gain limits | GR,max, GR,min | 35 dBi and −10 dBi; the clipping values in (3.10c) |
| Receive-gain envelope | AR, BR | 32 and 25; earth-station reference-pattern parameters of (3.10c), absorbed into Hu,s,v(t) |
| Boresight-limit threshold | ϵμ​ | 10−10​ |
| Rician fading | KR​ | 20 dB |
| Receiver system temperature | Ta, T0, NF, T | 150 K, 290 K, 1.2 dB, and 242.294 K |
| Fixed power | Pf(t)​ | Aggregate of RF-chain, baseband, and other fixed circuit terms |
| Segment-start transmit power | p0​ | 0.825 W, that is pmax/2; the starting value of every new served segment in Eq. (3.11). pmax/p0=2 gives every served segment a 3 dB gain budget; 3 dB is the beam’s own half-power contour (F=1/2 exactly at μ=2.07123, that is at θ=θ3dB/2, whose ground radius is Rb), so the size of the budget is fixed by geometry rather than chosen. The budget is measured against the segment-start angle θu,s,v(τu,s,v), not against the beam axis: the numerator of Eq. (3.12) is GT​θu,s,v(τu,s,v) and not G0, so the infeasibility test reads “the gain has fallen 3 dB since the link was taken up”, which coincides with “has left this cell” only when the segment starts on the axis |
| Per-beam RF-output limit | pmax​ | 1.65 W; the per-beam operating limit after back-off in Eq. (3.15a), and the threshold of the link-feasibility check. At V=39 a satellite radiating every beam at that limit draws 39×1.65=64.3 W, below the LEO maximum transmit power Pmax=50 dBm =100 W of Table I of HOBS [4], so the per-beam limit is consistent with the per-satellite power budget of the cited work; this study sets no per-satellite limit of its own (see the exclusions below Eq. (3.11)), and the comparison is a budget check only |
| PA maximum efficiency and output back-off | ξmax, BO | 0.35 and 5 dB; Eq. (3.15a), with saturated power psat=pmax10BO/10=5.218 W |
| Circuit power per active beam | Pcir​ | 0.338 W (300+19+14+5 mW, Table II of You et al. [26]); Eq. (3.16a) |
| Shared baseband power per satellite | PBB​ | 0.200 W [26]; Eq. (3.16a) |

Of the four losses of Eq. (3.10b), Eq. (1) of HOBS gives names but no values; this study substitutes the corresponding tables of the 3GPP non-terrestrial network report [17] for Lg, Lc and Ls, while Lf is computed directly from the slant range and the carrier frequency. This substitution is a modelling choice of this study rather than the original setting of HOBS. Ls and the Rician fading KR are the only two random terms in this model, and the experiments reproduce them from fixed random seeds.

式 (3.10b) 的四項損耗中，HOBS 式 (1) 只給出名稱而未給數值；本研究以 3GPP 非地面網路報告 [17] 的對應表格代入 Lg、Lc 與 Ls，Lf 則由斜距與載波頻率直接計算。此代換是本研究的建模選擇，不是 HOBS 的原始設定。Ls 與萊斯衰落 KR 是本模型僅有的兩個隨機項，實驗以固定亂數種子重現。

The active contract does not put minimum-rate inversion, beam/satellite caps, PA reference curves, or min/max projections into the EE main formula. If the legacy simulator uses such limits, they are execution history for this chapter and cannot rewrite Eqs. (3.11)–(3.17).

目前 active 契約不把最低速率反推、beam／satellite cap、PA 參考曲線或 min／max projection 放入 EE 主公式。這些限制若由 legacy simulator 使用，只能作為本章的執行歷史，不能反向改寫式 (3.11)–(3.17)。

If the existing numerical results were generated by the legacy runtime, they must be read as legacy provenance. Until runtime parity with the new contract is implemented and the experiments are rerun, the existing results are not claimed as empirical evidence for the new formulas.

本章既有數值結果若由舊版 runtime 產生，仍須以 legacy provenance 解讀；在新契約完成 runtime parity 並重新執行實驗前，不把既有結果宣稱為新版公式的實證。

| Legacy execution settings (Chapter 5 provenance only) | Symbol | Historical setting |
| --- | --- | --- |
| Minimum-rate threshold | Rm​ | 1 Mbit/s |
| PA reference output power | P0​ | 5.218 W; the old runtime’s name for the psat of Eq. (3.15a), with the same value |
| Atmospheric attenuation (legacy form) | χatm; Latm=3dkmχatm/(10hskm) | χatm=0.05. This form yields only 0.015 dB at zenith, an equivalent atmospheric thickness of 0.3 km, which does not match the magnitude of gaseous absorption at 20 GHz; it is superseded by the cosecant law of Table 5-2 |
| Satellite-total RF-output limit | Psat,max​ | 1013/10=19.953 W |
| PA reference efficiency (legacy) | η0​ | 0.35; superseded by ξmax |

Table 5-3. Method and evaluation settings

| Category | Symbol | Setting in this study |
| --- | --- | --- |
| Three-objective scalarization weights | Ω=(ω1,ω2,ω3)​ | (0.5,0.3,0.2)​ |
| Handover costs | (φ1,φ2)​ | 0.5 for an intra-satellite beam change and 1.0 for an inter-satellite handover |
| Network architecture | — | Every Q-network on both the main and catfish sides uses hidden layers of 100, 50, and 50 units |
| Learning and batch | B​ | The learning rate is a controlled variable swept over {0.01,0.003,0.001}; the batch size is B=128 |
| Exploration and target networks | ϵ, K | ϵ decays linearly from 1 to 0.01 over 2000 episodes; target synchronization every 50 episodes |
| Training horizon | E, H | 9000 episodes and 10 steps per episode |
| Main and catfish discounts | βM, βF | 0.90 and 0.99 |
| Energy-efficiency stratification | q1, q2, W | 0.50, 0.80, and 5000 transition bundles |
| Periodic intervention | ρI, [T1,T2] | Catfish share 0.30; interval sampled uniformly from 4 to 16 steps |
| Competitive reward | ηw​ | 1.0 |
| Objective scale constants | (c1,c2,c3)​ | (2471140.576, 1.0, 6); obtained from P3 and the analytic bound, frozen before training |

MODQN [2] reports a learning rate of 0.01. This study adopts neither default outright: the learning rate is treated as a controlled variable and swept over the three values above, and the value used for the main results is filled in once the experiments are complete.

MODQN [2] 報告的學習率為 0.01。本研究不逕自採用任一邊的預設值，而是把學習率列為受控變因並掃描上述三個值；主結果所採用的值於實驗完成後填入。

The frozen scale constants are (c1,c2,c3)=(2471140.576, 1.0, 6). Here c1 is the 95th percentile of r1,u over 12,000 served decision steps in probe P3, c2=φ2=1.0 is the analytic bound for the handover cost, and c3=6 is the rounded 95th percentile of |r3,u| from P3. They are fixed before training and shared by all update procedures; in the original units, the effective trade-off weight is ωj/cj. Post-training effective contributions or weight shares are not available until the trained policy exists and must not be inferred from these pre-training constants.

目前凍結的尺度常數為 (c1,c2,c3)=(2471140.576, 1.0, 6)。其中 c1 是 P3 探測在 12{,}000 個已服務決策步上的 r1,u 第 95 百分位數，c2=φ2=1.0 是換手成本的解析上界，c3=6 是 P3 探測中 |r3,u| 第 95 百分位數的取整值。三者在訓練開始前固定，並由所有更新程序共用；原始單位下的有效取捨權重為 ωj/cj。訓練後的有效貢獻或權重占比必須等訓練完成才可量測，目前尚無數值，不能由這些訓練前常數推論。

### Where the constraints bind

Whether the three constraints of this section are active is measured under the frozen scenario. The per-beam RF-output limit pmax fires on 113 of 12{,}000 decision steps (0.94%): the power the link requires exceeds the limit, the link is declared infeasible, and the user is unserved at that step.

在凍結情境下量測本節設定的三項約束是否作用。每波束射頻輸出上限 pmax 在 12{,}000 個決策步中的 113 步(0.94%) 上開火：該步所需的發射功率超過上限，該鏈路判為不可行，使用者於該步 unserved。

The peak link power among feasible steps is 1.6452 W, which uses 99.6% of the 3 dB gain budget. This is not a near miss; it is the limit truncating the distribution: among the steps declared infeasible, the required power has a median of 117.2% of the budget and a maximum of 214.8%. The tail of the required-power distribution reaches beyond twice the limit, and the feasible steps show only the cut left after it is removed.

可行步的峰值鏈路功率為 1.6452 W，已用掉 3 dB 增益預算的 99.6%。這不是「差一點超過」，是上限在削頂：被判為不可行的那些步，所需功率的中位數為預算的 117.2%、最大值 214.8%。所需功率的分布尾端伸到上限的兩倍以上，可行步只呈現它被切斷之後的切口。

These figures are bound to three things: Δt=30.08 s, the warm-start main arm, and the stay-if-possible reference policy. The policy matters most: the same settings under a random-masked reference policy give 7.60%, eight times as much, because that policy does not hold a link that has walked into good geometry. At Δt=1 s the outage rate is 0.0000, and with the warm start disabled, so that every episode boundary resets the served segment, it is also 0.0000 under the same policy (0.0016 under random-masked) with a peak reaching only 85.1% of the budget.

這組數字綁定於三件事：Δt=30.08 s、暖啟動主臂，以及 stay-if-possible 參考策略。策略的影響最大：在 random-masked 參考策略下同一組設定給出 7.60%，是前者的八倍，因為它不會保住已經走進良好幾何的鏈路。Δt=1 s 時 outage 為 0.0000；關閉暖啟動（每個 episode 的 served segment 都由邊界重置）時在同一策略下亦為 0.0000（random-masked 下為 0.0016），可行峰值只到預算的 85.1%。

The constraint fires under both sampling arms for the entry segment age. The main arm Uniform{0,…,H−1} gives a mean entry age of 4.56 steps and a firing rate of 0.94%; the sensitivity arm Uniform{0,…,5} gives 2.54 steps and 0.81%. The latter is the equilibrium age distribution of the measured segment length of 6 steps; the former is independent of the policy but samples ages that run old. The ages differ by a factor of 1.79 while the firing rates differ by only 1.16. The relation is a threshold rather than a linear one: raising the mean entry age from 0 to 2.54 steps takes the firing rate from 0.0000 to 0.0081, and raising it further to 4.56 steps adds only 0.0013. The constraint therefore does not fire because the main arm samples old ages. The main arm is the pre-registered main arm, and its firing rate is read as an upper bound.

進場段齡的兩條抽樣臂都會使約束開火。主臂 Uniform{0,…,H−1} 的平均進場段齡為 4.56 步，開火率 0.94%；敏感度臂 Uniform{0,…,5} 的平均為 2.54 步，開火率 0.81%。後者是量得段長 6 步所對應的平衡態年齡分布，前者與策略無關但年齡偏老。年齡相差 1.79 倍而開火率只相差 1.16 倍。此一關係是門檻型的而非線性的：平均段齡由 0 增至 2.54 步時開火率由 0.0000 升至 0.0081，再增至 4.56 步時只多 0.0013。因此約束會開火並非由主臂的偏老年齡撐起；主臂為預註冊之主臂，其開火率視為上界。

None of the three geometric mask terms binds in this setting, and Au(t) is 28 throughout. The mask and the power-feasibility rule are different mechanisms: the first decides what a user may select, the second decides whether the selected candidate becomes a connection.

三項幾何遮罩項在本情境下均不作用，Au(t) 恆為 28。遮罩與功率可行性是不同機制：前者決定使用者能選什麼，後者決定被選中的候選是否成為連線。

# 6. Conclusion

## 6.1 Summary

This study uses the Multi-Objective Deep Q-Learning Network (MODQN) as the baseline for handover in multi-beam low Earth orbit satellite networks and builds the Multi-Catfish Reinforcement Learning (MCRL) framework on that architecture. Its three objectives correspond to angle-aware system energy efficiency, handover cost, and load balancing. MCRL applies experience shaping and reward shaping during training. After training, only the main agent is retained, and each user continues to select the highest-valued feasible action independently.

本研究以多目標深度 Q 學習網路（MODQN）作為多波束低軌衛星換手的基準，並在此架構上建立鯰魚多目標強化式學習（MCRL）。三個目標分別對應角度感知系統能量效率、換手成本與負載平衡。MCRL 在訓練期間使用經驗塑形與獎勵塑形。完成訓練後只保留主代理，且每位使用者仍從可行候選中各自選出價值最高的動作。

## 6.2 Contributions

The main work can be summarized as follows.

本研究的主要工作可整理如下。

For angle-aware system energy efficiency, the first reward is the sum of the selected-link EE contributions, r1,u(t)=s,v​xu,s,v(t)ηu,s,v(t,θ). The off-axis angle first affects GT and then enters the wanted-link numerator directly as Hu,s,v(t)GT(θu,s,v) in Eq. (3.10) and the SINR in Eq. (3.13); the angle-aware RF power is determined by Eqs. (3.11)–(3.12), the same unique link SINR determines throughput, and Eqs. (3.15)–(3.17) form shared system power and EE. The main chain no longer uses target-SINR inversion, a second SINR, or concept-level caps/min/max. If the existing Chapter 5 numbers came from the legacy runtime, they require runtime parity with the new contract and a rerun before they can be treated as evidence for the revised formulas.

在角度感知系統能量效率方面，第一個獎勵使用所選鏈路 EE 貢獻的總和，r1,u(t)=s,v​xu,s,v(t)ηu,s,v(t,θ)。偏軸角先由式 (3.7) 影響 GT，再在式 (3.10) 以 Hu,s,v(t)GT(θu,s,v) 直接進入式 (3.13) 的 wanted-link numerator；角度感知 RF power 由式 (3.11)–(3.12) 決定，同一個唯一鏈路 SINR 再決定 throughput，並透過式 (3.15)–(3.17) 形成共同系統功率與 EE。本文不再以 target SINR、需求功率反推、第二套 SINR 或概念層 cap／min／max 描述主鏈；第五章的既有數值若來自 legacy runtime，須待新契約完成 runtime parity 並重跑後才能作為新版公式的實證。

For multi-objective catfish training, the main side and catfish side each contain three Q-networks corresponding to energy efficiency, handover, and load balancing; the three catfish-side networks share one environment trajectory and replay buffer. Energy-efficiency stratification, asymmetric discounting, and periodic mixed-batch intervention change the experiences and the within-episode return weighting used by the catfish side. The paired competitive reward changes only its first objective. After training, only the main side is retained.

在多目標鯰魚訓練方面，主代理與鯰魚代理各包含三個 Q 網路，分別對應能量效率、換手與負載平衡；鯰魚代理的三個網路共享同一條環境軌跡與經驗池。能效分層、非對稱折扣與週期性混合批次介入改變鯰魚代理所使用的經驗與回合內回報加權。配對競爭獎勵只調整鯰魚代理的第一個目標。訓練完成後只保留主代理。

## 6.3 Limitations and Future Work

System power includes the power amplifier, active-beam RF chains, satellite-shared baseband power, and beam-training and handover event energies. The current setting uses Etr=Eho=0, while other satellite hardware, signal-processing, and control power are outside the model. Future work can extend these power sources and reassess their effects on energy efficiency and method comparisons.

本文的系統功率包含功率放大器、啟用波束的射頻鏈、衛星共用的基頻功率，以及波束訓練與換手事件能量。目前設定 Etr=Eho=0，而其他衛星硬體、訊號處理與控制功率未納入模型。後續可擴充這些功率來源，並重新檢查其對能量效率與方法比較的影響。

The current model is defined for one simulated environment and includes idealized assumptions in both the energy-efficiency definition and the environment settings. The performance difference between MCRL and the baseline remains to be determined by complete experiments; later studies can examine different user counts, constellation conditions, and environments closer to practical systems.

目前模型以單一模擬環境為對象，能量效率定義與環境設定也包含理想化假設。MCRL 與基準方法的效能差異仍待完整實驗判定；後續可在不同使用者數、星座條件與更接近實際的環境中檢查方法的適用範圍。

# References

[1]	J.-H. Lee, C. Park, S. Park, and A. F. Molisch, "Handover Protocol Learning for LEO Satellite Networks: Access Delay and Collision Minimization," IEEE Transactions on Wireless Communications, Vol. 23, No. 7, pp. 7624-7637, Jul. 2024.

[2]	Y. Sun, Y. Zhai, W. Wu, P. Si, and F. R. Yu, "Handover for Multi-Beam LEO Satellite Networks: A Multi-Objective Reinforcement Learning Method," IEEE Communications Letters, Vol. 28, No. 12, pp. 2834-2838, Dec. 2024.

[3]	U. Ntabeni, B. Basutli, H. Alves, and J. Chuma, "Adaptive Handover Optimization in LEO Satellite Networks Using Energy-Aware Q-Learning," IEEE Open Journal of the Communications Society, Vol. 6, pp. 5657-5666, 2025.

[4]	S.-H. Chen, L.-H. Shen, K.-T. Feng, L.-L. Yang, and J.-M. Wu, "Energy-Efficient Joint Handover and Beam Switching Scheme for Multi-LEO Networks," IEEE Vehicular Technology Conference (VTC2024-Spring), pp. 1-7, Singapore, 2024.

[5]	M. O. K. Mendonça, E. Lagunas, J. Grotz, A. Pérez-Neira, and S. Chatzinotas, "Deep Q-Learning-Based Handover for Spectral Coexistence Between Feeder and User Links in LEO Satellite Networks," IEEE Open Journal of the Communications Society, Vol. 6, pp. 6777-6791, 2025.

[6]	H. Liu, Y. Wang, P. Li, and J. Cheng, "A Multi-Agent Deep Reinforcement Learning-Based Handover Scheme for Mega-Constellation Under Dynamic Propagation Conditions," IEEE Transactions on Wireless Communications, Vol. 23, No. 10, pp. 13579-13596, Oct. 2024.

[7]	J. Zhu, Y. Sun, and M. Peng, "Beam Management in Low Earth Orbit Satellite Networks With Random Traffic Arrival and Time-Varying Topology," IEEE Transactions on Vehicular Technology, Vol. 73, No. 9, pp. 13352-13367, Sep. 2024.

[8]	M. Song, J. Tian, H. Zhang, T. Xu, H. Hu, and T. Zhou, "Deep Reinforcement Learning Based Multi-Objective Handover for LEO Satellite Networks," IEEE Vehicular Technology Conference (VTC2025-Spring), pp. 1-5, Oslo, Norway, 2025.

[9]	T. Tajmajer, "Modular Multi-Objective Deep Reinforcement Learning With Decision Values," Federated Conference on Computer Science and Information Systems (FedCSIS), pp. 85-93, Poznań, Poland, 2018.

[10]	T. Basaklar, S. Gumussoy, and U. Y. Ogras, "PD-MORL: Preference-Driven Multi-Objective Reinforcement Learning Algorithm," International Conference on Learning Representations (ICLR), Kigali, Rwanda, 2023.

[11]	T. Schaul, J. Quan, I. Antonoglou, and D. Silver, "Prioritized Experience Replay," International Conference on Learning Representations (ICLR), San Juan, Puerto Rico, 2016.

[12]	B.-H. Ke, "CDRL: Catfish-Effect-Based Deep Reinforcement Learning for Energy-Efficient Joint Switching of RF Chains and Reflecting Elements in RIS-Aided 6G Networks," M.S. thesis, Dept. Comput. Sci. Inf. Eng., National Taipei Univ., Taipei, Taiwan, 2025.

[13]	M. Calvo-Fullana, S. Paternain, L. F. O. Chamon, and A. Ribeiro, "State Augmented Constrained Reinforcement Learning: Overcoming the Limitations of Learning With Rewards," IEEE Transactions on Automatic Control, Vol. 69, No. 7, pp. 4275-4290, 2024.

[14]	L. Agorio, S. Van Alen, M. Calvo-Fullana, S. Paternain, and J. A. Bazerque, "Multi-Agent Assignment via State Augmented Reinforcement Learning," Learning for Dynamics and Control Conference (L4DC), Proceedings of Machine Learning Research (PMLR), Vol. 242, pp. 1202-1213, Oxford, UK, 2024.

[15]	J. Holder, N. Jaques, and M. Mesbahi, "Multi-Agent Reinforcement Learning for Sequential Satellite Assignment Problems," AAAI Conference on Artificial Intelligence (AAAI), pp. 26516-26524, Philadelphia, PA, USA, 2025.

[16]	Q. Ye, B. Rong, Y. Chen, M. Al-Shalash, C. Caramanis, and J. G. Andrews, "User Association for Load Balancing in Heterogeneous Cellular Networks," IEEE Transactions on Wireless Communications, Vol. 12, No. 6, pp. 2706-2716, Jun. 2013.

[17]	3GPP, "Study on New Radio (NR) to Support Non-Terrestrial Networks (NTN)," 3rd Generation Partnership Project (3GPP), Technical Report TR 38.811, V15.4.0, 2020.

[18]	J. Yu and J.-H. Kim, "Analysis of SINR According to Elevation Angle in Earth Fixed Beam," Asia-Pacific Conference on Communications (APCC), pp. 633-634, Jeju, South Korea, 2022.

[19]	Y. Abudureheman, J. Chu, R. Song, X. Liu, W. Huangfu, and H. Zhang, "Resource Allocation in Multibeam LEO Satellite Systems Based on Beam Hopping and Frequency Reuse," IEEE Internet of Things Journal, Vol. 13, No. 1, pp. 167-180, Jan. 2026.

[20]	E. Kim, I. P. Roberts, P. A. Iannucci, and J. G. Andrews, "Downlink Analysis of LEO Multi-Beam Satellite Communication in Shadowed Rician Channels," IEEE Global Communications Conference (GLOBECOM), pp. 1-6, Madrid, Spain, 2021.

[21]	ITU-R, "Reference Radiation Pattern for Earth Station Antennas in the Fixed-Satellite Service for Use in Coordination and Interference Assessment in the Frequency Range From 2 to 31 GHz," International Telecommunication Union, Geneva, Switzerland, Recommendation ITU-R S.465-6, Jan. 2010.

[22]	3GPP, "Solutions for NR to Support Non-Terrestrial Networks (NTN)," 3rd Generation Partnership Project (3GPP), Technical Report TR 38.821, V16.2.0, 2023.

[23]	S. C. Cripps, RF Power Amplifiers for Wireless Communications, 2nd ed. Norwood, MA, USA: Artech House, 2006.

[24]	A. Piacibello, R. Giofrè, R. Quaglia, R. Figueiredo, N. Carvalho, P. Colantonio, V. Valenta, and V. Camarchia, "A 5-W GaN Doherty Amplifier for Ka-Band Satellite Downlink With 4-GHz Bandwidth and 17-dB NPR," IEEE Microwave and Wireless Components Letters, Vol. 32, No. 8, pp. 964-967, Aug. 2022.

[25]	A. Morello and V. Mignone, "DVB-S2—Ready for Lift Off," EBU Technical Review, No. 300, Oct. 2004.

[26]	L. You, X. Qiang, K.-X. Li, C. G. Tsinos, W. Wang, X. Gao, and B. Ottersten, "Hybrid Analog/Digital Precoding for Downlink Massive MIMO LEO Satellite Communications," IEEE Transactions on Wireless Communications, Vol. 21, No. 8, pp. 5962-5976, Aug. 2022.

[27]	V. Mnih, K. Kavukcuoglu, D. Silver, et al., "Human-Level Control Through Deep Reinforcement Learning," Nature, Vol. 518, No. 7540, pp. 529-533, 2015.